ENVIRONMENT RESEARCH

PROGRAMME

FINAL REPORT

#### Contract n°: ENV4-CT95-0161

Title :

RECONSTRUCTION OF VERTICAL OZONE DISTRIBUTION FROM

UMKEHR ESTIMATES (REVUE)

Coordinator : Sophie GODIN

Contractors :

C1 	Service d'Aéronomie CNRS (S. Godin)

C2 	LAP-AUTH (C. Zerefos,E. Kosmidis)

C3 	INM (E. Cuevas, A. Redondas)

C4 	CNR (L. Stefanutti, D. Guzzi)

Consultants :

R. Bojkov

J. DeLuisi

Key words : Stratosphere, Ozone, Umkehr, Trends, Variability, Algorithm

Reporting period : March 1996 – December 1999

TABLE OF CONTENTS

I. OBJECTIVE FOR THE CONTRACT PERIOD

##### II. SUMMARY OF MAIN RESULTS

#### The new Umkehr algorithm

  - A Priori information

  - Aerosol correction

  - Effect of the ancillary temperature data

###### Reprocessing of the Umkehr data

  - The new Umkehr data set

  - Discovery of shifts in the Umkehr records

###### Comparison with ancillary data

  - Comparison at OHP with lidar, sondes and SAGE II measurements

  - Extensive comparison with SAGE II

  - Brewer Umkehr – Ozone sondes comparison at Izaña

###### Statistical analysis of the Umkehr time series

###### Ozone trends evaluation

###### Conclusion on main achievements

###### Publications

###### III. Individual reports

IV. Annexes

I. OBJECTIVES FOR THE CONTRACT PERIOD

The general objective of REVUE is to produce a long-term homogeneous large scale data set of vertical ozone profiles based on the Umkehr method. Ozone measurements based on this method were performed for more then 35 years in several Dobson stations throughout the world. These measurements constitute a unique data set, likely to provide a good evaluation of the stratospheric ozone trends since long duration time series allow to accurately evaluate the natural processes influencing the ozone variability. The recent SPARC/WMO study (WMO, 1998) on the assessment of trends in the vertical distribution of ozone involved measurements from various satellite and ground based instruments, including Umkehr measurements performed from 1977 in some stations. It showed in general a good agreement among the instruments trend results in the middle stratosphere. However, important issues are still pending concerning the Umkehr trends, like the test and assessment of the new Umkehr algorithm proposed by Mateer and DeLuisi in 1992 (Mateer and DeLuisi, 1992) and the evaluation of the trends at the various stations where Umkehr measurements were performed, since the sixties for some of them. This is precisely the goal of the REVUE project, in which the proposed methodology is based on the following work plan:

- Test and improvement of the new Umkehr algorithm.

- Re-analysis of the raw Umkehr data with the final algorithm in order to produce a homogeneous data set

- Validation of these data with ancillary measurements (satellite, lidar and ozone sondes data)

- Statistical analysis of the Umkehr time series and ozone trend evaluation.

The main achievements of the REVUE project are now summarized in this final report.  Firstly, a large part of the project was devoted to the improvement and test of the 1992 Umkehr algorithm and to the quality control of the Umkehr data base. This work was achieved by the LAP together with Dr. J. DeLuisi and Dr. R. Bojkov as consultants. It focused on the Dobson Umkehr algorithm, which is the algorithm used for the retrieval of the ozone vertical distribution from measurements of the UV zenith solar radiation at twilight by Dobson instruments. The main points to be improved in the Umkehr algorithm were (i) the a priori information, (ii) the aerosol correction of the Umkehr data, (iii) the effect of ancillary pressure-temperature data on the retrieval. The second point itself was a demanding task since it required the implementation of a new radiative transfer vector code and the set up of a global aerosol data base. The IROE contributed to put together the aerosol data base, using for instance lidar data, and to determine the optical properties of the aerosol as a function of the wavelength.

It appeared in the course of the project that the homogeneization of the whole Umkehr data base was a long and fastidious task. This work was achieved by the LAP with the assistance of R. Bojkov. For the first time, the Umkehr records deposited at the World Ozone Data Center (WODC) in Toronto were carefully re-examined on an individual basis. Particular attention was given to the use of the Bass and Paur ozone absorption coefficients in each Umkehr record. The re-evaluation of the Umkehr record included also the check of the total ozone data at each station. Indeed, total ozone can be used as a priori information so the quality of these data is of prime importance for the Umkehr retrieval. Besides, it is meaningful to check the coherence of  the Umkehr trends in the various Umkehr layers and the total ozone trend at each station.

The quality control of the raw Umkehr data and the check of the movement of instruments at each station unfolded shifts in most of the Umkehr records. The effect of these shifts on the ozone retrieval were cautiously analyzed and a solution was proposed for their correction. But it was not possible in the course of this project to firmly establish a methodology for the correction of the shifts. Thus, the final Umkehr data set produced for the REVUE project is not shift corrected, but indication of the effect of the shifts on ozone trends are given in the report.

The new Umkehr data set was thoroughly validated against ancillary data and the improvements from the old Umkehr retrieval were emphasized. This work was performed extensively by the CNRS with the long-term ozone record at the Observatoire de Haute-Provence (OHP) and the SAGE II 15 year global record, and also at some stations by the LAP. A validation of Brewer Umkehr measurements against ozone soundings was also performed in the course of this project at Tenerife by the INM. The principle of the Brewer Umkehr measurements is very similar to the Dobson Umkehr one and since long term measurements of the ozone vertical distribution are performed by Brewer instruments throughout the world, it is hoped that these measuremements will benefit from the work achieved within the REVUE project.

The last part of the project was devoted to the statistical analysis of the Umkehr time series and the ozone trend evaluation with the new Umkehr data set. This work was achieved by the CNRS and the LAP with the assistance of Dr. R. Bojkov. The CNRS focused on the comparison of the seasonal sampling and seasonal evolution of ozone as retrieved by the various instruments implemented at OHP. This work was detailed in the first and second progress reports. It was also published in Guirlet et al., 2000. The study was performed on the 1992 Umkehr data set but the reprocessing of the Umkehr data did not change the main results obtained by this study. Finally, the statistical analysis of the ozone variability as a function of altitude was performed by the LAP with the assistance of Dr. Bojkov, on the new homogeneized Umkehr data set. The Umkehr ozone trends were also re-evaluated at selected stations and the main processes affecting the long term ozone variability were analyzed.

II. SUMMARY OF MAIN RESULTS

- The New Umkehr algorithm

The first part of the project was devoted to the improvement of the new Umkehr algorithm proposed by Mateer and DeLuisi in 1992. In the kick off meeting held in Paris, the main points to be studied were defined as (i) the a priori information, (ii) the aerosol correction and (iii) the effect of the temperature ancillary data on the Umkehr retrieval. This work is detailed in the LAP report and in Annex II. The present algorithm uses new latitudinal and seasonal dependant first guess profiles, a new radiative transfer vector code for the aerosols correction and updated latitudinal dependant temperature climatology.

1.1 A Priori information (First-Guess profile)

The Umkehr ozone profile retrieval algorithm is based on a best first-guess ozone profile and the covariance of the profile. The retrieval then seeks to effect a change to the first-guess profile from the information contained in the radiation measurement, with the covariance information telling the algorithm the most probable changes to the ozone profile. Therefore, the algorithm benefits from the best a priori information that can be acquired. In the course of the REVUE project, a new first-guess data set based on updated ozone sondes and SAGE II climatology was produced (see annex II). A study of the importance of a priori information on the Umkehr retrieval algorithm can also be found in Petropavloskikh et al., 2000.

1.2 Aerosol correction

A lot of effort was dedicated to the aerosol correction of the Umkehr data, since the long term records are perturbed in the lowermost and uppermost layers, by the volcanic aerosols originated in the stratosphere from the eruptions of several volcanoes such as El Chichon in 1982 and Mount Pinatubo in 1991. These eruptions injected high amounts of sulfur which ended up in the formation of intense volcanic clouds lasting about 3 years in the lower stratosphere (see WMO, 1999) for a review of the stratospheric volcanic aerosols. The work on the aerosol correction is detailed in the LAP and IROE reports and in Annex II. It is also published in DeLuisi et al. (2000). In the new algorithm, the procedure has been changed to adopt the more accurate radiative transfer vector code of Dave, instead of the scalar code from the same author. A large set of aerosol profiles from the SAGE satellite climatology, backscatter lidar and sun-photometry measurement were used to calculate aerosol error corrections as a function of stratospheric aerosol optical depth. A significant part of this study involved examination of the relationships between aerosol size distribution and extinction as well as other optical properties as a function of wavelength, in order to estimate the aerosol extinction at the UV wavelengths used in the Umkehr measurements. The main task of IROE during this period was to provide a collection of aerosol data to be included in the aerosol climatology of the new Umkehr algorithm. The data provided were the following :

- Stratospheric aerosol properties (backscatter and extinction coefficient) from the following lidar stations (Sodankyla, Florence, Lauder, Dumont d’Urville, OHP, Garmisch Partenkirchen, Mauna Loa, McMurdo, Eureka, Thule).

- Tropospheric aerosol optical properties from Florence and Thessaloniki. The tropospheric aerosol impact on the Umkehr values is evaluated to one order of magnitude less than the stratospheric one.

- Particles size distributions and refractive index in support of the study of general aerosol optical properties.

- Reprocessing of the Umkehr data

2.1 The new Umkehr data set

In order to achieve a coherent and homogeneous Umkehr data set, several controls were performed on the data deposited at the WO3DC archive in Toronto. The first control concerned the re-evaluation of total ozone records of the stations having long-term Umkehr measurements for ozone profile retrieval (see the LAP report and Annex I). In addition, rigid quality control of the Umkehr measurements was applied: from over 60,000 Umkehr profiles deposited initially in the WO3DC, 40,000 were finally selected for the REVUE statistical and long-term analysis. These are the profiles from stations with long enough (more than 20 years) records and of reasonable quality according to criteria imposed by statistical analysis. Bass and Paur (Bass and Paur, 1985) ozone absorption coefficients were used throughout this analysis. The stations selected for the final Umkehr analysis are displayed in the following table:

| # | Name | # Latitude | ## Longitude |
| --- | --- | --- | --- |
| 7 | Kagoshima | 31.60 | 130.60 |
| 10 | New Dehli | 28.65 | 77.22 |
| 12 | Sapporo | 43.05 | 141.33 |
| 14 | Tateno | 36.05 | 140.10 |
| 31 | Mauna Loa | 19.53 | -155.58 |
| 35 | Arosa | 46.78 | 9.68 |
| 40 | Haute-Provence | 43.93 | 5.70 |
| 67 | Boulder | 40.03 | -125.25 |
| 68 | Belsk | 51.84 | 20.79 |
| 152 | Cairo | 30.08 | 31.28 |
| 159 | Perth | -31.92 | 115.95 |
| 187 | Poona | 18.53 | 73.85 |
| 190 | Naha | 26.20 | 127.68 |
| 245 | Aswan | 23.97 | 32.45 |

2.2 Discovery of shifts in the Umkehr records

In the Umkehr method, the measurement correspond to the variation of the ratio of two zenith-sky UV intensities at selected wavelengths with solar zenith angle (SZA) changing from 60° to 90°. In the course of the project, a thorough review of the individual readings from the Dobson instruments for all the SZA was performed, with in parallel, the re-examination of the calibration records of all Dobson stations made during past inter-comparisons and total ozone re-evaluations. A number of "shifts" in the individual SZA data were identified and suggestions for homogenization of the original raw data accounting for the various "shifts" were made. This study is detailed in the LAP report.

- Comparison of the new Umkehr record with ancillary data

The new Umkehr data set was validated against other long term measurements such as the SAGE II satellite measurements taken at the vicinity of each station and in the case of the Haute-Provence Observatory, the ozone sondes and stratospheric ozone lidar measurements which started in 1984 and 1985 respectively (see the CNRS report). Launched in 1984, SAGE II provides the longest satellite record of ozone profiles measurements. Depending on the quality and temporal sampling of the Umkehr record and the SAGE II temporal sampling at each station, the number of coincident measurements ranges from 15 in New Dehli to 450 in Arosa. In most stations with a reliable long term Umkehr record, the new algorithm shows a better agreement with SAGE in layer 3, with a (Umkehr – SAGE) bias of –10%. In layer 4, the new Umkehr retrieval provide lower ozone amounts which result in a small negative bias of around –5% of the Umkehr data with respect to SAGE II. The best agreement is generally found in layer 5 to 7, the optimal range of the Umkehr algorithm in terms of information content. Layer 8 is also part of the optimal range but a positive bias of the Umkehr with respect to SAGE II is generally found. In the stations with less reliable Umkehr records, the new algorithm tends generally to improve the agreement with the SAGE II data.

In parallel ozone sondes measurements campaigns were organized in Tenerife in order to compare the local Brewer Umkehr and the sondes measurements. Brewer Umkehr measurements use a similar Umkehr method but with a Brewer instead of a Dobson spectrophotometer. The objectives of these campaigns were to initiate the same work with the Brewer Umkehr retrieval algorithm as was done with the Dobson Umkehr one. In particular, several version of the Brewer Umkehr algorithm (Wave5-old, Wave5 and Wave6) were carefully analyzed and validated against ECC vertical profiles. The comparisons showed that the most simple Umkehr code, the early Wave5 version (1991 code) provides reasonably good results. Wave5 and Wave6 (1994 codes) do not seem to improve Umkehr profiles and, on the contrary, might introduce some uncertainties. Besides, a new ozone first-guess obtained from the ECC climatology for 1-7 layers was incorporated to the Wave5 (1991) code, improving significantly the results. An analysis of possible Umkehr quality control indexes was also performed. The cloud monitoring, the number of iterations, the RMSRES and the 303.6/323.2 ratio against the zenith angle provide useful information about the Umkehr success. The main conclusion of the comparisons was that the Brewer-Umkehr algorithm provides reasonably good results in layers 4, 5 and 6 for the description of the ozone seasonal variation in these layers and also for the trend detection.

- Statistical analysis of the Umkehr time series and trend evaluation

The last part of the REVUE project involves the statistical analysis of the ozone variability at each station and the re-evaluation of ozone trends. The statistical calculations and trends that have been produced from the revised Umkehr data set, cover two periods: 1978-97 and 1964-99 (see the LAP report and annex III). The 14-months long periods following the El-Chichon and Mt. Pinatubo eruptions were excluded from the analysis. Statistical analysis of the ozone variability showed that the absolute and relative variability reach their maximum during winter-spring in the altitudes below 24km. Two-thirds of the total ozone variability is contributed by these layers. Layer 5 (24-29km) although containing about 20% of the total ozone shows the least fluctuations, no trend and contributes only ~7% to the total ozone variability. The deduced trends above 30 km confirm strong ozone decline since mid-1970s of over 6% per decade without significant seasonal differences. In the northern mid-latitude stations in the 15-24km layer, the decline is nearly twice as strong in the winter-spring season but much smaller in the summer and fall. Comparisons of the integrated ozone loss from the Umkehr measurements with the total O3 changes in the same periods at the stations with good records are in complete concurrence. The altitude and latitude dependence of the long-term geophysical signals like QBO (3 to 6% below 25km) and the solar (up to 2% above 35km) are detected. Adding two more years of data in 1998 and 1999, characterized by a relatively high total ozone as compared to the previous years significantly changed (reduced by nearly one half) the negative trend rates in the lower stratosphere but did not change significantly the rates in the upper stratosphere. Finally, a comparison was made between the results obtained in the 40°N-52°N latitude band in the WMO/SPARC study (WMO, 1998) and the revised Umkehr data in Belsk, Boulder and OHP, over the same time period. The results are presented in figure 1:

Figure 1 : Long term ozone trends (1) from the last SPARC/WMO Assessment (WMO report 43, 1998) at 40°-52°N and (2) combined from the revised Umkehr records at Boulder, OHP and Belsk.

##### Conclusion

As a conclusion, the REVUE project has provided the unique opportunity to revise one of the most important data set for the evaluation of the stratospheric ozone long term trend. The Umkehr algorithm was completely re-evaluated and longstanding issues on the Umkehr retrieval were examined, like the influence of the a priori information in the algorithm and the aerosol correction of the retrieved ozone values. For this latter task, a global aerosol climatology was set up, using all the relevant data available, such as the long term aerosol lidar measurements performed by European institutions. This work on the Umkehr retrieval will certainly benefit to the scientific communauty working on the inversion methods for atmospheric measurements, since nadir satellite measurements like those from the SBUV instrument for instance, are based on the same physical principle as the Umkehr measurements. In particular, the last REVUE meeting was open to members of the satellite communauty, which triggered scientific collaboration between the groups working on these measurements.

The Umkehr record itself was thoroughly checked. This was a long and tedious task, taking into account the numerous persons involved in the data archiving since the sixties. Artefacts in some of the Umkehr records were uncovered and solutions were proposed for their correction. The validation of the new Umkehr data with ancillary satellite and ground-based measurements, showed generally a better agreement mainly in the lower stratosphere, for most of the reliable long term records.

Finally, we fulfilled our objectives on ozone statistical analysis and trend re-evaluation. On the latter point, several temporal periods were studied, showing that the addition of the most recent Umkehr data decrease the ozone downward trends. It is likely that the year-to- year dynamical variability of the stratosphere will induce such fluctuations in the ozone trends as the stratospheric chlorine starts to decrease, and the new homogeneous Umkehr data set will be essential for the study of the future evolution of the stratospheric ozone.

- References

Mateer, C.L. and J.J. DeLuisi, 1992, A new inversion algorithm, J.Atm. And Terr. Phys., 54, 537-556.

Bass A. M., and R. J. Paur, Proc. Quadriennal Ozone Symposium, Halkidiki, C. Zerefos and A. Ghazi, Reidel, Dordrecht, 1984.

WMO report 43, Global ozone research and monitoring project, Assessment of trends in the vertical distribution of ozone, 1998.

WMO report 44, Global ozone research and monitoring project, Scientific assessment of ozone depletion, 1999.

V. DEVIATION FROM TECHNICAL ANNEX AND REASONS

VI. PUBLICATIONS

The main REVUE achievements will be presented at the Quadriennal Ozone Symposium in July 2000, with 5 contributions related to:

- The global presentation of the REVUE project by Godin et al.

- The re-evaluation of ozone trends from Umkehr data by Bojkov et al.

- The aerosol error correction in the Umkehr retrieved ozone values by DeLuisi et al..

- The re-evaluation of Raw Umkehr data by Kosmidis et al..

- The assessment of uncertainty in the Umkehr retrieval by Petropavloskikh et al.

A CD Rom has been produced, containing the following items :

- This final report and annexes

- The final Umkehr data set

- The new Umkehr algorithm

List of publications:

Bojkov Rumen D., Vertical ozone distribution characteristics deduced from 44,000 re-evaluated Umkehr Profiles from the 1957-1999 period, Intern. O3Symp, Sapporo, Japan, 2000

Deluisi J.J., I. Petropavlovskikh, A. Stevermer, D. Theisen, Aerosol Error Corrections in Umkehr Retrieved Ozone Profile Record, Intern. O3Symp, Sapporo, Japan, 2000

Godin S., C. Zerefos, E. Kosmidis, K.Tourpali, E. Cuevas, D. Guzzi, L. Stefanutti, R.D. Bojkov, J.D. DeLuisi, I. Petropavlovskikh, D. Theisen. Reconstruction of Vertical ozone distribution from Umkehr Estimates REVUE, Intern. O3Symp, Sapporo, Japan, 2000

Guirlet, M., Keckhut, P., Godin, S. And Megie, G., Description of the long-term ozone data series obtained from different instrumental techniques at a single location: The Observatoire de Haute-Provence (43.9 °N, 5.7°E), accepted at Ann. Geophys., 2000.

Kosmidis E., R. D. Bojkov, J. DeLuisi, I.V. Petropavlovskikh, D. Theisen, and C. Zerefos, Re-evaluation of Raw Umkehr data, Intern. O3Symp, Sapporo, Japan, 2000

Petropavlovskikh I.V. J. J. DeLuisi, D. Theisen, R. D. Bojkov and E. Kosmidis., Umkehr Retrieval Algorithm: Assessment of Uncertainties, Intern. O3Symp, Sapporo, Japan, 2000a

Petropavlovskikh, I.V., J. J. DeLuisi, D. Theisen, R. D. Bojkov and E. Kosmidis. "Detection of Shifts in the Umkehr N-value Radiance Records" submitted to GRL 2000b.

VII. ANNEXES

Six annexes are joined to this report:

- Annex I: Reanalysis of the total ozone record

- Annex II: Improvements and tests of the Umkehr algorithm

- Improved First-Guess Ozone profiles

- Effect of the First-Guess in SBUV and Umkehr comparisons

- Comparisons of Umkehr updated First-Guess Profile retrieval with SAGE

- Upgrade to full forward radiative transfer model in the 1992 algorithm

- Umkehr profile trend analysis study

- Stratospheric Aerosol study

- Annex III: Trend analysis

- Seasonal issues

- Seasonal weighting

- Autocorrelation

- Trend term

- Effect of neglecting solar, QBO and other terms

- Annex IV: Figures related to the statistical analysis of the ozone variability

- Annex V: Figures related to the trend analysis

- Annex VI.: Analysis and validation of the different Brewer-Umkehr codes

PART B: INDIVIDUAL REPORT

Reporting period: March 1998 - December 1999

Contractor: 	Laboratory of Atmospheric Physics, Aristotle University of Thessaloniki

Leading scientist: Christos Zerefos

Consulting: Rumen D. Bojkov, John DeLuisi, Irina Petropavlovskikh, Vitali Fioletov, Dave Theisen, and Amy Stevermer

Scientific Staff: Evagelos Kosmidis, Kleareti Tourpali

Address: Thessaloniki 54006

Telephone: +30-31-998041 - 998156

Fax: +30-31-248602

E-mail: zerefos@auth.gr

# I. OBJECTIVES.

- Continuation of total ozone re-evaluation at the selected Umkehr stations with corrections due to instrumental calibrations and incorporating the Bass and Paur absorption coefficients. Careful collection, quality control (including use of N-values) and analysis of available data. (Task 1)

- Analysis of the sensitivity to temperature changes. (Task 2)

- Exploration of the influence of first guess profiles to the processing and creation of new a-priori profiles for the algorithm. (Task 3)

- Aerosol Corrections for the retrieved data. (Task 4)

The last phase included the following tasks:

- A well documented analysis of the vertical ozone distribution changes during the past 30-35 years. (Task 5)

- Detailed comparisons with profiles derived from satellite observations. (Task 6): To be presented by CNRS

- Thorough analysis of the spatial and temporal variability of vertical ozone distribution trends and of known ozone oscillations in the vertical including the QBO and ENSO components. (Task 7)

# II. Main results obtained

According to the allocation of tasks between partners the following have been accomplished:

## Re-evaluation and quality control of the past ozone data (Task 1)

- This task involves the careful quality control and re-evaluation of total ozone records of several Dobson stations having long-term Umkehr zenith-sky radiance measurements for ozone profile retrieval. In addition, rigid quality control of the Umkehr measurements was applied. Dr. Bojkov(WMO) in collaboration with Dr. DeLuisi(NOAA), Dr. Vitali Fioletov (AES) E. Kosmidis(LAP) have completed this Part of the project. This analysis completes the work done for re-evaluating the total ozone records at sixteen (16) stations possessing long term regular Umkehr observation programs (see Annex 1).

- The quality control of Umkehr records from the 16 stations under study is now considered completed. A final set of data has been collected from either individual stations or from WO3DC directly. This data set consists of 12 Solar Zenith Angles N-Values for each Umkehr measurement at every one of the 16 Umkehr stations.

- From over 60,000 Umkehr profiles deposited in the WO3DC (mentioned in the original proposal) 50,000 have been used for the REVUE analysis. These are the profiles from stations with long enough (more than 20 years) records and of reasonable quality according to criteria imposed by statistical analysis and explained later in the main part of this report. From these 50,000 profiles available some (~13,000) were eliminated due to the following reasons:

- In some stations there were very long interruptions (more than 5 years) of observations making it impossible to derive any useful information for trend analysis (Bojkov et. al., 1990).

- Even after the quality control at some stations the retrieved profiles showed great variability and large errors based on criteria drawn out, from the algorithm itself, (DeLuisi 1995, Matteer and DeLuisi 1992). See Figures 1-15.

- Specific Profiles have been left out if it was found that even after the quality control there was an unusual ozone value (out-layer) frequently related to an abnormal N-value, or for some other obvious reason (e.g. wrong format of the data which gave wrong profiles).

- Finally, monthly mean data were calculated only if more than 4 profiles per month have been accepted based on the above-mentioned quality control procedures. If there were 3 profiles, which did not deviate from the 2sigma envelope for the given month, these were also accepted.

After eliminating of erroneous and doubtful measurements, the status of the available acceptable data is shown in Table 1:

TABLE 1

| Name, Latitude | # of Obs Used | # of Obs Available |
| --- | --- | --- |
| Belsk (51.84) | 1321 | 1945 |
| Arosa (46.78) | 8595 | 10883 |
| Haute Provence (43.93) | 2641 | 3160 |
| Sapporo (43.05) | 1169 | 1348 |
| Boulder (40.03) | 2652 | 3142 |
| Lisbon (38.77) | 1281 | 1885 |
| Tateno (36.05) | 5504 | 6127 |
| Kagoshima (31.60) | 1286 | 1594 |
| Cairo (30.08) | 3392 | 3238 |
| New Delhi (28.65) | 938 | 1385 |
| Naha (26.20) | 668 | 841 |
| Varanasi (25.32) | 657 | 1168 |
| Aswan (24.37) | 1641 | 2756 |
| Mauna Loa (19.53) | 3915 | 5368 |
| Poona (18.93) | 519 | 915 |
| Perth (-31.92) | 3496 | 3864 |
| Total | 38675 | 50083 |

Bass and Paur (B-P) (Bass and Paur, 1985) ozone absorption coefficients have been used throughout this analysis of all 50,000 total ozone records, as recommended by the Ozone Commission and WMO in 1992. There were cases where the station operators have been correcting the records by themselves before submission to WMO-WO3DC.In these cases measures of caution have been exercised to avoid redundant correction of the data. It was discovered that some of the original records (N-values) provided by WO3DC were corrected for B-P twice. This was due to the fact that even though the stations were providing B-P corrected data sets, WO3DC had no knowledge of this, thus correcting the data again. In order to check the correct B-P status of the data each profile separately had to be examined and be compared with the retrieved (with WO3DC’s version of the algorithm) ozone profiles deposited in the WO3DC data bank to realize the systematic differences and decide with certainty whether the corrections were applied once, twice or none and act accordingly so that a complete set of “finalized true” original N-value profiles have been collected.

In this vain of checking the data, it was very helpful to compare with the total ozone data used in the WMO Ozone Assessment -1998. Before any change was made, we were checking if the data agree with the data used in the  Assessment. Thus, the REVUE data set is probably the first Umkehr data set in which the best available total ozone information is used for evaluation of the vertical ozone distribution.

The data deposited in WO3DC were separated based on the B-P corrections into two subsets; One, before January 1992, where the Vigroux coefficients had been used and another one after January 1992, when stations were requested to use the B-P coefficients. For the stations that needed corrections the value of 0.9743 has been used to convert data from Vigroux to B-P scale in accordance to the IO3C-WMO change of absorption coefficients published in the "red books" of the WO3DC. Some stations had already corrected their complete records, (Boulder, Haute Provence) both for the B-P and for instrumental errors, therefore, providing a very stable, complete and coherent data set. The two Egyptian stations (Cairo and Aswan) have re-deposited their re-evaluated total ozone data in WMO-WO3DC only in the early part of 2000. Results of this report are based on the most complete records available from stations, which at some stations start in 1957. JMA (Japan Meteorological Agency) has alone reported and deposited their re-evaluated data after correcting for some instrumental calibrations identifying problems with the optical parts of the wedges.

The data used in REVUE are extending till mid-1999. However, inclusion of the data from last two years (especially winter-springs) introduced a very strong tail effect at the end of the record distorting the calculations. This effect comes from the fact that in 1998 and 1999 in the Northern middle latitudes the ozone amount was on the average relatively high (in some seasons close to the pre-1976 averages). Therefore some of the trend analyses were done for period finishing with December1997 and compared with the trends derived from complete data set. This is an issue on which more analysis is needed.

### Comments on few station records

Station 007. Kagoshima is a station with many problems in its N-values record. From Table 2 it is clear that the Dobson type instruments at this station has been changed at least x7 times. Considering possible differences in the optical alignments of the individual instruments it is not a surprise that this data record exhibits many shifts. The following Table (2) provided by JMA illustrates the times of instrument changes.

### Movements of instruments.

The first three columns are the year, month and day of first operation. The next three columns is the same information for the last date of the period of operation, and in the last column is given the number of the instrument used in that station.

TABLE 2

SAPPORO

YYYY MM DD YYYY MM DD INSTRUMENT NUMBER 1958 1 18 1959 6 9 5704 1959 6 10 1961 10 9 5706 1961 10 10 1964 12 15 5704 1964 12 16 1968 10 31 5705 1968 11 1 1976 2 29 5704 1976 3 1 1981 12 31 5702 1982 1 1 1985 2 28 5703 1985 3 1 1989 5 31 5704 1989 6 1 1994 8 31 128 1994 9 1 1998 5 11 129 1998 5 12 NOW 126

TATENO

YYYY MM DD YYYY MM DD INSTRUMENT NUMBER 1955 6 1 1959 3 3 52 1959 3 3 1959 4 12 5706 1959 4 13 1975 12 31 52 1976 1 1 1977 7 10 116 1977 7 11 1978 12 31 5601 1979 1 1 1984 10 2 116 1984 10 3 1984 12 31 5704 1985 1 1 1989 5 31 116 1989 6 1 1989 7 31 127 1989 8 1 1993 12 31 116 1994 1 1 NOW 125

NAHA

YYYY MM DD YYYY MM DD INSTRUMENT NUMBER 1974 4 1 1978 2 28 5703 1978 3 1 1982 12 31 5705 1983 1 1 1986 12 31 5706 1987 1 1 1988 12 31 5705 1989 1 1 1994 2 28 129 1994 3 1 1996 9 30 126 1996 10 1 NOW 127

Also due to the fact that there is an active volcano in a close distance to the station volcanic SO2 sometimes interferes with measurements of total ozone. Moreover, lack of information on SO2 concentrations made it impossible to use regular method (se discussion above) to correct total ozone data for the BP ozone absorption cross-sections. Also the comparison with the Assessment -1998 data showed that total ozone during the period from Nov. 63 to Jul.67 and from Jan 70 to Mar 87 has to be increased by 9 DU.

Station #010: New Delhi data before 1974 are not used due to various unresolved instrument calibration problems. All the data before 1992 were in Vigroux scale and had to be corrected. Data after 1992 were in the B-P scale already.

Station records of #012(Sapporo), and #190(Naha) have been re-evaluated by the JMA. According to the information from JMA on the movement of instruments (see Table 2) it was possible to detect and correct some of the N-value discrepancies. The #014(Tateno) data were found to be of a good quality; therefore, no extra correction was necessary. It should be mentioned that in the WO3DC archives the original data of all of the Japanese stations had been converted twice for B-P scale through the end of year 1991, a fact that was corrected with the recent re-submission of the data.

Data from station #035(Arosa) had been re-evaluated and re-submitted by the operators. However, the new data seems to have some problems in the late 1980’s. All the data were in B-P scale.

Station #68(Belsk) data in the WO3DC was in Vigroux scale through the end of 1991, and in B-P scale thereafter. All data were corrected to be in B-P scale.

Stations #152(Cairo) and #245(Aswan) have been re-evaluated and re-deposited by the operators in early 2000. Then, we performed an extensive review including comparisons to the Ozone Assessment data. Some corrections to the Aswan data were based on N-values record assessment (in concurrence with the operators). All records are in a good shape now after conversion of data before January 1992 to B-P scale.

Stations #31(Mauna Loa), 40(OHP), 67(Boulder) and 159(Perth) are using a NOAA-automated Dobsons. All data have been uniformly processed and deposited to the WO3DC. Data appear to be of a good quality. Data at Station #159 before year 1985 have had to be converted to the B-P scale.

All of the above stations had data available until August 1999.The re-processed data are now available in the REVUE file. For various reasons, like major instrumental errors, lack of data homogeneity, missing time-periods essential for trend analysis, and other similar reasons, some stations have to be left out from the ozone trend analyses and profile inter-comparisons. We strongly recommend exercising great caution while working with these records. Among stations requiring special attention there are #74 (Varanasi), 82(Lisbon), 187(Poona) and 192(Mexico).

### Additional criteria for acceptance of an Umkehr profile.

An additional selection criterion was applied to improve quality of the data used in the analysis: the measurement was accepted if the retrieved profile RMS error was less than 5%. This criterion is discussed analytically by Matteer and DeLuisi (1992).

Another criterion that was used is based on the fact that the ozone profile is retrieved using optimum statistical estimation method (Strand and Westwater, 1968) that requires an iterative process. In other words, based on the first guess profile algorithm attempts to approach the best result by iterating the inversion procedure up to the point where there is no improvement possible (judged by the residual error estimate). Therefore, profiles undergone more than 4 iterations have been left out from the final data set for statistical analysis and have been flagged as such in the final data set. The resulting final time series for the available stations can be seen in Figures 1-15.

The time series of the Umkehr retrieved ozone profiles consist of monthly mean values of ozone amounts in each layer. The profiles have been filtered based on the requirement of 4 profiles per month in order to increase representativeness of the monthly means and minimize noise in trend analysis. However, if using 4 profiles per month was impossible due to the lack of available data, then 3 profiles were considered for monthly averages if and only if data were inside of the standard error envelope for the seasonal mean value.

For the equatorial zone due to very low ozone variability, 3 profiles should be sufficient for establishing a monthly mean.

Another procedure that was followed in order to get better regional and/or latitudinal understanding of the vertical ozone distribution and its changes  was  to group the available Umkehr data according to latitude zones. So, for example European stations such as Belsk, Arosa and OHP are grouped together. Also some of the sub-tropical stations (Aswan, Naha, Mauna Loa, New Delhi) were combined.

Another approach for better presentation of the results in statistical and trend analysis was to separate data by seasons (see annex III by Dr. Bojkov). In this aspect we used both the three (natural) seasons “model” and the four climatological season “model”. In the three seasons model we use DJFMA (for December, January, February, March and April), MJJA (for May, June, July and August) and SON (for September, October and November). The natural “model” is better reflecting the annual changes in the middle latitude stations.

The final set of useful monthly mean data from each of the stations can be seen in the following Table 3. The maximum length of a record could include 480 months (1957-1999). The numbers given in Table 3 correspond only to trend analysis of the full period, while some shorter periods also have been used (1978 - 1997 and 1964 - 1999). It was decided not to use the years before 1964 although the data are available, since there are only a few stations that have data into this period and there is a relatively low number of observations. Also the period of 1978-1997 has been chosen to provide ground for comparisons with other studies that use the same period which is a reference for the time where the strongest ozone decline appeared.

Maximum Number of months used in the calculation

| Station (Latitude) | Number of Months |
| --- | --- |
| Belsk (51.84) | 264 |
| Arosa (46.78) | 373 |
| Haute Provence (43.93) | 151 |
| Sapporo (43.05) | 275 |
| Boulder (40.03) | 202 |
| Tateno (36.05) | 363 |
| Kagoshima (31.60) | 279 |
| Cairo (30.08) | 173 |
| New Delhi (28.65) | 144 |
| Naha (26.20) | 171 |
| Aswan (24.37) | 124 |
| Mauna Loa (19.53) | 132 |
| Perth (-31.92) | 216 |

Table 3

## N-values analyses

Before we move further to the analysis it would be useful to mention that the Umkehr analysis provides information on the vertical ozone distribution in ten layers. This layer structure can be seen in the following table (4) (Matter and Deluisi 1992)

| Layer Number | Pressure at the bottom (mb) | Approximate Height at the bottom (km) |
| --- | --- | --- |
| 1 | 1000 mb | 0 |
| 2 | 250 | 10.3 |
| 3 | 125 | 14.7 |
| 4 | 62.5 | 19.1 |
| 5 | 31.2 | 23.5 |
| 6 | 15.6 | 28.0 |
| 7 | 7.81 | 32.6 |
| 8 | 3.91 | 37.5 |
| 9 | 1.95 | 42.6 |
| 10 | 0.9 and above | 47.9 |
| TABLE 4 |  |  |

During the process of taking an Umkehr measurement, the variation of the ratio of two zenith-sky ultra-violet (UV) intensities (ZSI) at selected wavelengths with solar zenith angle (SZA) changing from 600 to 900 is measured. The 12 angles used are: (60, 65, 70, 74, 77, 80, 83, 85, 86.5, 88, 89, 90). The logarithm of this ratio is traditionally called an N-value.

A specialized examination of the historical Umkehr N-value measurement record revealed shifts in the data. These shifts could be mainly attributed to instrumentation calibration or replacement activities at an observing station. Calibration procedures may change optical properties of individual Dobsons. Moreover, these changes (observed as shifts in the data) may vary with SZA The standard procedure is used to eliminate the instrumental and extraterrestrial constants from Umkehr measurements by subtracting the Umkehr measurement at 60-degree solar zenith angle (or the smallest SZA available) from measurements at other SZAs. However, it does not necessarily cancel shifts in many of the cases. Therefore, the shift error would propagate into the retrievals as shifts in ozone profile and could affect an ozone trend analysis if thorough control is not exercised. A preliminary approach for detecting a shift and allowing for its correction is developed and evaluated for its potential to improve the Umkehr record.

In the previous studies shifts seen in the long-term series plots of the Umkehr retrieved ozone profiles are visually identified and corrected by statistical intervention method prior to trend analysis [e. g. Reinsel et al., Weatherhead et al., 1997]. While the approach to correct time-series shifts by interventions appears to work commendable well with the trend analysis, a more satisfying approach is to examine the Umkehr radiance data to acquire insight into the nature of the shifts. The characteristic patterns of the changes in Umkehr measurements need to be studied and a physical explanation must follow. With this information, it might be possible to correct the Umkehr record of each station to eliminate shifts in the modified data set. Yet, the profile information should suffer minimal interference from the correction process. When an instrument is changed or calibrated, the station’s instrumental characteristics are most likely to be changed. To account for this, a provision is made in the Umkehr ozone profile retrieval algorithm to remove the dependence on instrumental and extraterrestrial constant. This is done by subtracting the Umkehr measurement, taken at the smallest SZA from the measurements taken at other SZAs. The method works only if the optical characteristics of an instrument are stable over the solar zenith angle range of measurement during its long-term service. However, such conditions are not always met. Therefore, there is a possibility of introducing shifts in the long-term Umkehr record at a station. This report describes an exploratory investigation of the shift problem and provides examples of its effects on Umkehr record.

Dates of N-value shifts are more clearly identified by examining time-series of the ratio of N-value at 60 SZA (N60) to total ozone (omega). The N60 measurement is the log of ratio of UV radiances at two wavelengths. When light propagates through atmosphere, the direct-beam UV radiation is attenuated by the bulk of ozone layer described by Beer’s law before it is scattered by tropospheric molecules and large particles where troposphere acts as a diffuser of the direct beam. Therefore, there is a linear relation between N60 and total ozone. To a very good approximation, N60 data can be theoretically represented as a linear function of the total column ozone: N60= a+ b* total ozone. The value of ‘a’ is determined by the ratio of extra-terrestrial solar irradiance at the C-pair wavelengths, the instrument wedge calibration, and the ratio of zenith-sky scattered radiation in the absence of ozone. The contribution to ‘a’ from Rayleigh scattering is estimated to be close to -8 N-values. The value of ‘b’ is determined primarily by the differential ozone absorption cross-section of the C-pair, but it also is weakly dependent on ozone profile at 60 SZA. However, this is not so at the larger SZAs where the shape of ozone profile affects the N-values, and is precisely the effect that provides the profile information in the Umkehr measurement.

For some of the stations an attempt has been made to identify the shifts and correct them in the trend analysis. These stations are Tateno, Sapporo, Naha, Aswan, Kagoshima, Arosa, OHP, Boulder, Belsk. For these stations a complicated and elaborate procedure has been followed. Firstly the shifts were identified by examining the N-Values time series. (As in the following figure for Tateno).

Tateno, N60 Values (N-Values at 60 degrees Zenith angle)

Nevertheless from a figure like that it is very difficult for one to find where exactly there is a need for an intervention. Baring in mind also that the Umkehr algorithm actually uses a normalised N-value which make many of the abnormalities to disappear, we can use the ratio of the N-Value to total ozone which will uncover the disturbances by magnifying them. So we use figures like the following:

Tateno, Ratio of the N-Value at 60 degrees to the total ozone for that profile.

In these types of figures the differences are more profound and subsequently much easier to be identified. After this tedious and elaborate semi-manual identification of “shifts” we had to somehow assess the importance of that shift. For this we used a statistical reasoning in which we calculated the shift for each Umkher zenith angle and we were identifying if the shift existed in all of SZA measurements. If it did exist, after the normalization to N60 (subtract N-60 from all the angles) this shift would cancel out and would not be a problem. If the shift was found only to exist and was of different magnitude in some angles then the approach was to identify if the difference of the shift in one angle with the shift in N60 was more than 2N units, which is a measure of almost 5% change in the resulting profile. In that case the shift was corrected statistically by using a best fit line to the N-Value time series and thus “bringing” the series to the desirable correct level. A paper by Kosmidis et.al., will be presented at the Ozone Symposium-2000 at Sapporo, Japan. In table 6 we see the identified shifts for all the stations and with bold are the shifts that are considered to be important. Also in figures 23-25 we see the resulting N-Value time series if one tries to correct the identified shifts. These figures are for Tateno, Arosa and Kagoshima for three selected solar zenith angles. Correcting the N-value measurements rather than the retrieved data is very convenient since shifts in Umkehr radiance measurements are more clearly identifiable compared to shifts found in retrieved ozone profiles. Also by correcting the N-values we provide the algorithm with a better quality input for better retrievals. In this work a first attempt has been made to identify and correct these shifts. All the available data are being deposited to WMO WO3DC and are available to the scientific community.

| Name | Profile | Shifts | Dates |
| --- | --- | --- | --- |
| Kagoshima | 1520 | 13 | 8/60, 7/61, 11/63, 1/65, 1/70, 1/72, 8/74,1/77, 9/80, 7/82,3/84, 1/86, 9/89 |
| New Delhi | 1313 | 14 | 10/63,1/67,1/68, 10/68, 7/70, 6/74 |
| Sapporo | 1251 | 5 | 6/74, 3/76, 3/85, 6/89, 9/94 |
| Tateno | 5711 | 7 | 6/63, 8/74, |
| Arosa | 10040 | 8 | 4/58, 1/67, 1/72, 1/88 |
| OHP | 2824 | 5 | 6/86,6/88,1/93 |
| Boulder | 2752 | 6 | 1/79, 1/80, 6/83, 1/91 |
| Belsk | 1786 | 7 | 7/74,1/78,6/79,12/82,9/90,1/93,9/95 |
| Varanasi | 1163 | 16 | 1/65, 1/66, 7/68, 1/70, 7/71, 7/72, 9/74, 1/77, 7/80, 1/84, 7/88, 10/89, 8/92, 6/95 |
| Lisbon | 1766 | 13 | 1/69, 1/70, 1/71, 1/72, 1/73, 4/74, 8/78, 8/86, 6/88, 9/90, 6/92, 6/94 |
| Cairo | 605 | 4 | 4/92 |
| Perth | 3080 | 7 | 1/75, 11/75, 8/84, 6/91, 12/92 |
| Poona | 828 | 7 | 10/76, 7/79, 6/83, 6/85, 6/86, 7/92, 1/95 |
| Naha | 681 | 6 | 1/83, 1/87, 1/89, 1/94, 8/96 |

Table 4

Many of the shifts were not corrected based on the decision that their effect on the retrieved ozone profile was not very strong (less than 5 %). The correction procedure was also not applied if statistically identified shift value was not statistically significant and, therefore, if applied, would only increase noisiness of the data. Nevertheless in some of the records (Naha, Aswan, Sapporo) the corrections were used and the results can be seen in the figures TD9, TD10, TD11.

In Aswan, for example, after careful examination of the record and after consultation with the operators it was decided to correct the data set during the years 1991 and 1992 due to known problems in the instrument. Also in the records of Naha single correction was made due to the change of instruments. After January 1989 Dobson #129 has been used, but before that Dobson #5705 (since Feb.1978 except 1983-86 when #5706 was used) was used. Both instruments, #5705 and #5706, have problems with internal light scattering. Therefore, retrieved ozone appears to be always too low for the layers VI and above ("private communication by Dr. M. Shimizu, 31 May 1983”).

All these interventions are illustrated in the above mentioned figures using the trends derived from each case. Correction of shifts in measurements may decrease the errors in the ozone profile trend detection, so with this work we conclude the fact that there is a reliable way of identifying the shifts. Nevertheless it was decided not to correct them since there is a need for a more reliable approach to the method of correction. Such method should include a complete analysis of the Dobson calibration procedure as well as absorption coefficient study that could ensure better continuity in the calibration record for both total ozone and Umkehr measurements (Petropavlovskikh et. al. 1999).

## Temperature and Aerosol corrections (Task 2, and 4).

Task 2 involves the analysis of the sensitivity of the method to temperature changes. As a result of REVUE study, major improvements in trend analysis and inter-comparisons of ozone profiles were achieved. The contribution of John DeLuisi and his group (SRRB/NOAA/USA) was essential and it is summarized in Annex II including also their contribution for tasks 3 and 4.

Data from the National Center for Atmospheric Research/National Center for Environmental Prediction Re-analysis Project and from the Naval Research Laboratory were used to compile a climatology of monthly and zonally averaged temperature profiles for the globe. The temperature profiles used till now were average profiles for 30-degree bands while the profiles used now are in 10-degree latitudinal bands and are seasonally averaged. The resulting profiles have a vertical resolution of 1 km from the surface to 120 km altitude. The NMC temperature profile climatology has been used to update the accuracy of the Umkehr retrieved ozone profiles. Change of the temperature profile from the standard atmosphere global average profile affects estimates of ozone absorption coefficient and, thus, forward model calculation of N-values in the retrieval algorithm. The difference in the retrieved ozone profiles with and without application of temperature climatological profiles is found to be within +/- 5 percent for Northern middle latitudes, and depends on season and altitude. Results of this study could provide temperature corrections for ozone profiles derived at individual Dobson stations. The temperature corrections can be applied to the ozone profiles retrieved using standardized Umkehr algorithm in the same fashion as aerosol corrections.

The aerosol corrections were determined from a first order, standardized set of calculations described by DeLuisi et al. (1996). Dave's vector radiative transfer (RT) code was used for theoretical calculations of N-values because, as mentioned above, the Dave’s scalar RT code, used in the past, lacked the necessary accuracy (Petropavlovskikh et al., 1996). Standard ozone profiles and a large set of aerosol profiles from the SAGE satellite climatology, LIDAR and sun-photometry measurement were used to calculate a set of aerosol error corrections as functions of stratospheric aerosol optical depth.

The aerosol corrections have been produced by John DeLuisi (SRRB/NOAA), Irina Petropavloskhki and Amy Stevermer (CIRES/CU), and have been applied to the retrieved ozone profiles providing corrected new time-series for trend analysis and profile comparisons with other measurements. The aerosol corrections are given per Umkehr layer, in latitude zones of 10 degrees starting from 20 to 60. The aerosol corrections for selected layers for each latitude zone can be seen in Figures 16-19. Α more detailed presentation of the aerosol error work can be found in Annex II which also includes the later advances made by J.Deluisi and his group. A paper by Petropavlovskikh et.al. will be presented in the Ozone Symposium 2000, at Sapporo, Japan.

As part of the Umkehr ozone record analysis, satellite, LIDAR, and other ground-based measurements are utilized to compile a long-term climatology of stratospheric aerosol optical depth needed to compute retrieved ozone profile errors caused by stratospheric aerosol interference. The LIDAR data that were used can be seen in tables 5,6.

Table 6a.  Locations and dates of lidar observations

| Location | Dates | Wavelength (nm) |
| --- | --- | --- |
| Boulder, CO (40N) | 10/84-12/88 | 694 |
| Hampton, VA (38N) | 06/82-11/94 | 694 |
| Mauna Loa, HI (19.5N) | 01/74-12/88 | 694 |
| Tateno, Japan (36N) | 04/82-12/83, 06/91-04/94 | 694 |
| Haute Provence, France (44N) | 06/91-08/94 | 532 |
| Camaguey, Cuba (22N) | 01/92-12/93 | 1060 |
| Brasilia, Brazil (22S) | 03/72-12/94 | 589 |
| Garmisch, Germany (47.5N) | 01/77-12/85, 02/94-07/95 | 532 |
| Frascati, Italy (42N) | 05/82-04/84, 01/91-09/94 | 532 |

Data from (Boulder, CO; Hampton, VA; Mauna Loa, HI; Camaguay, Cuba; Tateno, Japan; Frascati, Italy; Florence, Italy; Haute- Provence, France; Garmisch, Germany; Brasilia, Brazil) had been used in addition to SAGE II and broadband pyrheliometer and photometer measurements from Mauna Loa, HI; Tucson, AZ; and Japanese stations at Nemuro, Miyako, Matsumoto, Ashizuri, and Shionomisaki.) Florence data are for only 5 months in 1996, and during that period we also have SAGE data. We concentrated mostly on northern latitudes where we had data going back to 1960s. In the Southern latitudes we had only few aerosol profile data records (Brazil, 1972-1994; Lauder, few months in 1994 and 1996; McMurdo fall of 1990 and 1991; and DDU, 1989-1995) with only Brazil data extending beyond SAGE II time frame. What is more important, there are really few Dobson records in Southern hemisphere, and even less have long-term record (Aspendale, Perth, Lauder, Natal, to name a few).

Table 6b.  Locations and dates of pyrheliometric transmission and turbidity data.

| Station | Instrument | Latitude | Dates | Wavelength range |
| --- | --- | --- | --- | --- |
| Tucson, AZ | Near incidence pyrheliometer | 32N | 1956-1997 | 300 – 4000 nm |
| Mauna Loa, HI | Near incidence pyrheliometer | 19.5N | 1958-1977 | 200 – 3000 nm |
| Nemuro, Japan | Pyrheliometer | 43.3N | 1953-1993 | 300 – 4000 nm |
| Miyako, Japan | Pyrheliometer | 39.6N | 1953-1993 | 300 – 4000 nm |
| Matsumoto, Japan | Pyrheliometer | 36.3N | 1953-1993 | 300 – 4000 nm |
| Ashizuri, Japan | Pyrheliometer | 32.7N | 1953-1993 | 300 – 4000 nm |
| Shionomisaki, Japan | Pyrheliometer | 33.4N | 1953-1993 | 300 – 4000 nm |

A significant part of this study involves examination of the relationships between aerosol size distribution and extinction as well as other optical properties as a function of wavelength with emphasis in the UV region. These relationships provide empirical means for estimating aerosol extinction at various wavelengths (including UV-B wavelengths) from a single LIDAR backscatter value, or from an extinction value at a different wavelength. A total of 134 in situ-measured size distributions, reported in the literature by several investigators, were utilized to develop these relations. The results suggest the existence of a bounded and predictable domain for aerosol optical properties, and indicate distinct differences between the volcanic and background aerosol regimes. The methods allow compilation of 320-nm stratospheric aerosol optical depth climatology, for years from 1953 to 1997.

The following figure shows the resulting stratospheric aerosol optical depth time series for each 10-degree latitude zone from 10 N to 50 N.

Optical depth data at 320 nm for all latitude bands from 60 S to 60 N are available, though the time series for southern hemisphere regions are largely derived from SAGE II and do not have the time period coverage of the data shown here. The optical depths are derived from integrated aerosol profiles above the tropopause boundary.

Signatures of the volcanic eruptions of Agung, Fuego, El Chichon, and Mt. Pinatubo are clearly visible in the time series. Although the monthly and zonal averaging used to create these time series may obscure some of the transport detail, the evolution of the stratospheric aerosol remains clearly visible. The decay times for the large eruptions of El Chichon and Mt. Pinatubo, respectively, appear to be about of the same magnitude for all latitude bands in the northern hemisphere. The optical depth maxima tend to be unchanging as the aerosol is rapidly transported as a pulse to the higher latitudes in the wintertime hemisphere. The spikes corresponding to higher aerosol optical depths are associated with these pulses of aerosol being transported northward in the wintertime stratospheric circulation.

Use of multiple data sets of stratospheric aerosol observations (such as aerosol lidar data from Florence, Garmish, Obsevatoire Haute Provance, and Dumont D'Urville provided by IROE) allows development of long-term stratospheric aerosol climatology. This climatology can be used for corrections to Umkehr and other types of remote sensing retrievals that are affected by volcanic aerosols, and also has applications for climate research. Our analysis has utilized a large, but by no means exhaustive, number of measurements from various locations and investigators. Our goal was to select a large group of data sets, based on location, data availability, and length of record, for estimating climatology of stratospheric aerosol depth at UV-B wavelengths. The resulting aerosol corrections can be seen in figures 16,17,18 were generally the corrections are positive in higher layers and negative in the lower ones.

## First guess corrections (Task 3)

Task 3 involves the exploration of the influence of the first guess profile to the Umkehr retrievals. The Umkehr ozone profile retrieval algorithm is dependent on a best first-guess ozone profile and the a priori ozone profile error covariance matrix.  The retrieval then seeks to implement a change to the first-guess profile from the information contained in the radiation measurement, with the covariance information telling the algorithm the most probable changes to the ozone profile.  Therefore, it benefits an algorithm to have the best a priori information that can be acquired. The algorithm 1992 (Mateer and Deluisi, 1992) has been using a priori information based on first four years (1978-1982) of global SBUV measurement combined with ozone-sonde data (Bhartia et al, Atmospheric Ozone, Proc. Ozone Symposium, Halkidiki. publ. by Reidel, 1984). Therefore, at the REVUE kick off meeting in Paris (1996) it was decided that more work needs to be done to incorporate more realistic first guess ozone profiles in the algorithm in order to improve the retrieval which will reflect positively on analyses of ozone variations and trends. John DeLuisi group has produced new first guess profiles based on updated ozone-sonde and satellite (SAGE II) ozone climatology. The final version of the first guess profiles, as well as the specific aerosol correction, is now available to the project and has been used for the final re-evaluation of all the Umkehr records.

An independent test of the success of the application of the new aerosol correction and first guess ozone profiles to the newly retrieved Umkehr ozone profile in the altitudes below 30 km was achieved by comparing ozonesonde measurements with Umkehr retrieved profiles. These comparisons can be seen in Figure 20-22 for stations 012, 014, 067, for which there were data available for both Umkehr and ozonesondes on the same days. The resulting profiles are with cases where more than 200 coincident profiles were available. Generally the new algorithm provides better agreement (almost 10 to 30 % better), (Petropavlovskikh et. al. Ozone Symposium 2000, Deluisi et.al. ozone Symposium 2000, Godin et. al. Ozone Symposium 2000) although the error bars in ozonesondes are pretty large (overlapping with the curves from the revised and the 1992 algorithm) so drawing conclusions out of these graphs is extremely difficult, with the ozonesonde data in all layers, especially for some stations.

# III. Presentation of the final time series.

In Figures 1-15, one can see the resulting time series for each of the stations that were finally used. In these graphs the decrease of the spread, compared with the previous algorithm retrievals (REVUE midterm report) can be seen. This is due mainly to the use of better aerosol corrections, better first guess profiles and better temperature profiles. From the figures of time series one can draw useful conclusions on the quality of each station’s record. The missing periods and the wide spread of data can be easily spotted in stations with long but of uncertain quality data, like Kagoshima which especially because of the near-by volcano exhibits this behaviour. Also using these time series and the quality considerations expressed earlier, we concluded that the data from Lisbon, New Mexico, Edmonton, Varanasi and Poona could not be used for any long-term analysis. On the other hand the records of Sapporo, Arosa, Tateno, Boulder, OHP, Mauna Loa, Aswan can be considered as of  good quality, something that can also be concluded from statistics drawn from these data and shown further in this report.

# IV. Presentation of results for the statistics.

In Table SA1, (Part A, Statistics) we see that the main changes in the ozone partial pressure (and/or content), when the total amount is stratified by 30 m-atm-cm increments from <280 to >400 m-atm-cm, is observed in layers II (68%), III (57%) and IV (45%). In layer V suddenly the change is only one third or half as big as 22%. This is the layer, which at these latitudes has a transitional regime from the photochemistry governed above 30 km to the dynamical circulation that affects ozone below 24 km.

The absolute values of the standard deviations of the layers means are highest in layers II, III and IV. (Part A, Statistics) Figure SA3, shows cross-section of the relative variability).

The ozone max calculated with the new algorithm-99 is in layer IV as determined also by sondes (~21km). With the previously used algorithm-92 the ozone maximum at these latitudes was always in layer V (too high compared to the ozone soundings results!)

Above layer V, the ozone partial pressure, as well as the ozone content, (see average in matm-cm in line 1 of Table SA3, Part A Statistics) rapidly declines, so do the ozone changes in the same layer between <280 and >400 groups. For example in layer VI the change is 40 matm-cm (19%); in layer VII is 22 matm-cm (also 19%); in layer VIII is 10 matm-cm (16%); in layer IX is 3.4 matm-cm (18%) and in layer X only 1.5 matm-cm (17%) change between the two extreme groups of <280 and >400.

The main changes from season to season are observed in the lower stratosphere Figure SA1, (Part A, Statistics) The partial pressure in Layer V is nearly constant during the three natural seasons it is 115 nb in wsp, 128 in summer and 110 nb in autumn. Expressed in matm-cm the ozone content is correspondingly 19, 20 and 21.5 % of the seasons total. In the layers VI-X the fractional content of ozone from the total in % is about 22% in wsp, 25% in summer and 26.5% in autumn.

From Table SA3 (Part A, Statistics) we see that for the relative variability of ozone content in layer V is with the smallest (10% only) fractional contribution to the overall ozone variability. The layers II, III, and IV together contribute 57% to the overall ozone variability; therefore we cannot ignore what is happening in these three layers Figure SA4, (Part A, Statistics). The fractional ozone content in layers II+III+IV is nearly half (47%) of the total ozone and each contributes about 20% to the total variability. Layers VI+VII contain less than 20% (19.5%) and contribute only 14% to the total variability, the top layers VIII+IX+X contain only 4.2% and contribute also the same ~4% to the total variability. A paper by Bojkov et.al. will be presented at the ozone symposium 2000 at Sapporo, Japan.

Cross-section of the month-to-month ozone changes (see Figures SA5) show that the greatest positive change (increase) is observed between Dec-to-Jan-to-Feb in the lower stratosphere (below layer V anyway!). The rapid decline is observed in May-to-June-to-July. That is why the natural seasons from the point of view of the ozone annual course and changes should be DJFMA, MJJA and SON. (see also discussion on Bojkov et.al. 1990).

# V. Presentation of results for the trends.

## Long term variability of ozone from revised Umkehr profiles.

The statistical models for calculation of trends in total ozone and its vertical distribution are discussed in details by WMO, [1998] {Ozone Report #43 and references therein}. Here we use the model of Fioletov, which has demonstrate good estimates in

the SPARC analysis. All the trend model runs have been done by Dr. Vitali Fioletov (AES, CANADA).

Let  represent monthly mean ozone values at a certain level. The statistical model for  is of the form:

= (Annual cycle) + (Seasonal trend) + (Solar effect) + (QBO effect) + Noise

Two types of statistical models were used to estimate trends in Umkehr data in this study.  Limited number of data points and large gaps in records of some stations make it difficult to estimate trends individual months and a seasonal trend model was used instead.  Reinsel (et al, [1999]) suggested a model of this type for Umkehr data analysis. In our study the seasonal statistical model is of the form:

yt=μ+St+i=14βiIi;tRt+γ1Z1;t+η1Q1;t+η2Q2;t+Nt                             (1)

|  | Ozone mean level |
| --- | --- |
| St | Seasonal component consisting of sinusoidal terms of periods 12, 6, 4, and 3 months |
| Ii;t | Indicator series for season i of the year; i.e., 1 if the season corresponds to month i of the year, and 0 otherwise. The four seasons are winter (December-February), spring (March-May), summer (June-August), and fall (September-November) |
| i | Trend in Dobson units/year in month i of the year |
| Rt | Linear ramp function; equal to (t-to)/12. For the time period of 1964-1999, it is taken to be a ramp function equal to zero for t < to, where to corresponds to 12/1969, and then (t-to)/12 for t  to. For the period 1978-1999, to corresponds to 12/1977 |
| Z1;t | Solar 10.7 cm flux (centered by subtraction of 120 units from the original data and normalized by division by 120), with 1 the associated coefficient |
| Q1;t, Q2;t | Normalized (with mean=0 and =1) equatorial wind at 30 and 50 mb series with 1 and 2 the associated coefficients |
| Nt | =AR(1) autoregressive: Nt = Nt-1 + et where et is an uncorrelated series |

A monthly trend model was used to analyze seasonal dependence of the trends in details and to calculate trends for “natural” ozone seasons.  This model estimates the trends for 12 individual months and is widely used for total ozone trend calculations [WMO Ozone Assessment, WMO Ozone Project Report #44], [Bojkov et al, 1995].  We used the monthly model only for stations with continuous records and few missing months:

yt=i=112μiIi;t+i=112βiIi;tRt+γ1Z1;t+η1Q1;t+η2Q2;t+Nt                                (2)

with the definitions:

| i | Ozone mean in month i, i = 1…12 |
| --- | --- |
| Ii;t | Indicator series for month i of the year; i.e., 1 if the month corresponds to month i of the year, and 0 otherwise |
| i | Trend in Dobson units/year in month i of the year |
| Rt | Linear ramp function; equal to (t-to)/12. For the time period of 1964-1999, it is taken to be a ramp function equal to zero for t < to, where to corresponds to 12/1969, and then (t-to)/12 for t  to. For the period 1978-1999, to corresponds to 12/1977 |
| Z1;t | Solar 10.7 cm flux (centered by subtraction of 120 units from the original data and normalized by division by 120), with 1 the associated coefficient |
| Q1;t, Q2;t | Normalized (with mean=0 and =1) equatorial wind at 30 and 50 mb series with 1 and 2 the associated coefficients |
| Nt | =AR(1) autoregressive: Nt = Nt-1 + et where et is an uncorrelated series |

Monthly weights were used in the statistical models to account for different ozone variability in different seasons as described by WMO, [1998].

All parameters were calculated in absolute units (DU of DU/year), however the final results are presented in percents or percents per decade.  To do so, the seasonal trends in DU/year in the model (1) were divided by the ozone mean level () plus the average of the seasonal component (St) values at three months of the season.  Similarly, the seasonal trends in percent per decade were calculated from the model (2) by averaging monthly trends in DU/decade within a season and dividing by the seasonal average of ozone means (i). The amplitudes of solar-related and QBO-related terms were also expressed in percent by division by the ozone mean level () in the model (1) and by the average of i., i=1,…,12 in (2).

| Table 7 - Belsk |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| level | DJF | e_djf | MAM | e_mam | JJA | e_jja | SON | e_son | year | e_year | a | err_a | b | err_b | c | err_c | parameter | N | mmean |
| 1 | 0 | 4.9 | -2.9 | 2.2 | -0.7 | 1.6 | -2.7 | 2.2 | -1.6 | 1.4 | 5.2 | 2.9 | 0.3 | 1.3 | -0.9 | 1.3 | 0.02 | 280 | 35.19 |
| 2 | 2.9 | 7.6 | -2.8 | 2.7 | -0.6 | 2.2 | -2.7 | 2.4 | -0.8 | 2.2 | 5.1 | 3 | 0.5 | 1.4 | -1.4 | 1.3 | 0.06 | 280 | 35.04 |
| 3 | 5.1 | 8 | -2.2 | 2.7 | -0.5 | 2.1 | -2.2 | 2.4 | 0.1 | 2.4 | 4.4 | 3.1 | 0.8 | 1.4 | -1.7 | 1.4 | 0.07 | 280 | 54.17 |
| 4 | 0.3 | 5.4 | -1.6 | 2.1 | -0.6 | 1.5 | -1.6 | 1.6 | -0.9 | 1.6 | 1.6 | 2.2 | 0 | 1 | -1.3 | 1 | 0.08 | 280 | 71.8 |
| 5 | -3.2 | 3.3 | -0.8 | 1.4 | -0.2 | 1 | -1.4 | 1.2 | -1.4 | 1 | -0.2 | 1.6 | -0.8 | 0.7 | -0.6 | 0.8 | 0.08 | 280 | 60.06 |
| 6 | -3.2 | 3 | -1.8 | 1.4 | -0.6 | 1 | -1.4 | 1.8 | -1.7 | 1 | 0.8 | 1.9 | -0.9 | 0.8 | -0.4 | 0.9 | 0.02 | 280 | 38.48 |
| 7 | -1.4 | 3.5 | -2.4 | 2.1 | -1.2 | 1.7 | -1.2 | 1.9 | -1.5 | 1.2 | 1.7 | 2.7 | 0.2 | 1.2 | -1.1 | 1.2 | 0.17 | 280 | 21.08 |
| 8 | -0.2 | 4.1 | -1.8 | 2.9 | -1.1 | 2.6 | -1.1 | 2.3 | -1.1 | 1.5 | 1.6 | 3.4 | 0.4 | 1.6 | -1.4 | 1.6 | 0.36 | 280 | 9.58 |
| 9 | 1.1 | 3.6 | -1.4 | 2.3 | -0.7 | 1.8 | -0.9 | 1.6 | -0.4 | 1.3 | 1.4 | 2.3 | 0.2 | 1.1 | -1.2 | 1.1 | 0.31 | 280 | 3.27 |
| 10 | 1.1 | 3.6 | -1.5 | 1.7 | 0 | 1 | -1 | 1.2 | -0.3 | 1.1 | 2 | 1.7 | 0.2 | 0.8 | -1.2 | 0.8 | 0.13 | 280 | 1.48 |
| 11 | 4.3 | 7.8 | -2.4 | 2.7 | -0.5 | 2.1 | -2.4 | 2.4 | -0.2 | 2.3 | 4.8 | 3.1 | 0.7 | 1.4 | -1.6 | 1.4 | 0.07 | 280 | 89.25 |
| 12 | 2.5 | 6.5 | -2.1 | 2.3 | -0.6 | 1.8 | -2 | 2 | -0.5 | 2 | 3.5 | 2.6 | 0.4 | 1.2 | -1.5 | 1.2 | 0.07 | 280 | 161.05 |
| 13 | 2.3 | 6.3 | -1.9 | 2.3 | -0.6 | 1.7 | -1.8 | 1.9 | -0.4 | 1.9 | 2.9 | 2.5 | 0.4 | 1.2 | -1.5 | 1.2 | 0.07 | 280 | 126 |
| 14 | 0.2 | 3.7 | -1.7 | 2.6 | -1 | 2.3 | -1.2 | 2 | -0.9 | 1.4 | 1.6 | 3 | 0.3 | 1.4 | -1.3 | 1.4 | 0.36 | 280 | 14.35 |
| 15 | 0.2 | 4.1 | -1.7 | 1.7 | -0.6 | 1.1 | -1.8 | 1.2 | -1 | 1.2 | 2.6 | 1.7 | 0.1 | 0.8 | -1.1 | 0.8 | 0.11 | 280 | 329.93 |
| 16 | 0.1 | 4 | -1.5 | 1.6 | 0.1 | 1 | -1.5 | 1.2 | -0.7 | 1.2 | 2.8 | 1.7 | 0.1 | 0.8 | -1.1 | 0.8 | 0.09 | 280 | 331.91 |

| Table 8 - Arosa |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| level | DJF | e_djf | MAM | e_mam | JJA | e_jja | SON | e_son | year | e_year | a | err_a | b | err_b | c | err_c | parameter | N | mmean |
| 1 | -3.1 | 1.7 | -3.3 | 1.7 | -2.4 | 1.2 | -2.2 | 1.7 | -2.8 | 0.8 | 2.3 | 1.7 | -0.4 | 0.9 | -0.5 | 0.8 | 0.15 | 392 | 28.85 |
| 2 | -1.5 | 1.5 | -2.4 | 1.8 | -1.5 | 1.3 | -1.2 | 1.6 | -1.8 | 0.8 | 1.3 | 1.5 | -0.1 | 0.7 | -0.5 | 0.7 | 0.1 | 392 | 29.39 |
| 3 | -1 | 1.9 | -1.9 | 1.7 | -0.9 | 1.2 | -0.3 | 1.6 | -1.1 | 0.8 | 1.3 | 1.6 | -0.2 | 0.8 | -0.6 | 0.8 | 0.1 | 392 | 49.16 |
| 4 | 0.2 | 1.6 | -1.3 | 1.3 | 0.5 | 0.9 | 0.9 | 1.3 | 0 | 0.7 | 1.1 | 1.4 | -0.6 | 0.7 | 0 | 0.7 | 0.06 | 392 | 73.17 |
| 5 | -0.2 | 1.3 | -1.2 | 1.3 | 0.5 | 0.9 | 0.8 | 1 | 0 | 0.6 | 0.8 | 1.3 | -0.9 | 0.6 | 0.4 | 0.6 | 0.24 | 392 | 63.34 |
| 6 | -2.9 | 1.2 | -2.6 | 1.1 | -2 | 0.8 | -1.8 | 1 | -2.3 | 0.5 | 0.9 | 1.1 | -0.7 | 0.6 | 0.2 | 0.6 | 0.24 | 392 | 40.75 |
| 7 | -5.4 | 1.3 | -5.2 | 1.2 | -4.3 | 1 | -4.7 | 1.1 | -4.9 | 0.6 | 1.3 | 1.3 | -0.2 | 0.7 | 0 | 0.7 | 0.27 | 392 | 23.31 |
| 8 | -4.3 | 1.8 | -5.1 | 1.8 | -3.8 | 1.6 | -4.2 | 1.6 | -4.4 | 0.9 | 1.7 | 2 | -0.1 | 0.9 | 0.2 | 1 | 0.32 | 392 | 10.54 |
| 9 | -3 | 1.4 | -3.9 | 1.5 | -2.4 | 1.2 | -2.4 | 1.2 | -2.9 | 0.7 | 1.6 | 1.4 | -0.2 | 0.7 | 0.1 | 0.7 | 0.27 | 392 | 3.5 |
| 10 | -2.4 | 1 | -2.6 | 1 | -1.5 | 0.6 | -1.2 | 0.9 | -1.9 | 0.5 | 1.5 | 0.9 | -0.3 | 0.5 | -0.2 | 0.5 | 0.15 | 392 | 1.56 |
| 11 | -1.2 | 1.7 | -2.1 | 1.7 | -1.2 | 1.3 | -0.6 | 1.6 | -1.4 | 0.8 | 1.3 | 1.6 | -0.1 | 0.8 | -0.6 | 0.8 | 0.09 | 392 | 78.56 |
| 12 | -0.6 | 1.6 | -1.7 | 1.4 | -0.4 | 1 | 0.2 | 1.4 | -0.7 | 0.7 | 1.3 | 1.4 | -0.3 | 0.7 | -0.4 | 0.7 | 0.07 | 392 | 151.73 |
| 13 | -0.3 | 1.6 | -1.5 | 1.4 | -0.1 | 1 | 0.4 | 1.4 | -0.4 | 0.7 | 1.3 | 1.4 | -0.4 | 0.7 | -0.3 | 0.7 | 0.06 | 392 | 122.32 |
| 14 | -3.8 | 1.6 | -4.7 | 1.6 | -3.3 | 1.4 | -3.5 | 1.4 | -3.8 | 0.8 | 1.7 | 1.7 | -0.1 | 0.8 | 0.1 | 0.8 | 0.32 | 392 | 15.6 |
| 15 | -1.5 | 1.1 | -2.1 | 1 | -1 | 0.6 | -0.7 | 0.9 | -1.4 | 0.5 | 1.3 | 1 | -0.4 | 0.5 | -0.2 | 0.5 | 0.1 | 392 | 323.42 |

|  |  |  |  |  |  |  | Boulder - Table 9 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| level | DJF | e_djf | MAM | e_mam | JJA | e_jja | SON | eson | year | e_year | a | err_a | b | err_b | c | err_c | parameter | N | mmean |
| 1 | -0.8 | 5.3 | -4.6 | 4.6 | -3.3 | 3.5 | -1.3 | 4.2 | -2.8 | 2.2 | 5 | 2.5 | 0.7 | 1.5 | -0.9 | 1.5 | 0.27 | 202 | 25.02 |
| 2 | 0.7 | 4.5 | -5.2 | 4.6 | -2.8 | 3.8 | -0.2 | 4.6 | -2.2 | 2.3 | 3.6 | 2.2 | 0.6 | 1.3 | -0.8 | 1.2 | 0.18 | 202 | 24.52 |
| 3 | 1.2 | 5 | -5.6 | 4.5 | -3 | 3.4 | -0.4 | 3.6 | -2 | 2.2 | 3.9 | 2.1 | 0.9 | 1.2 | -1.1 | 1.2 | 0.18 | 202 | 41.49 |
| 4 | -0.2 | 4.5 | -4.2 | 3.7 | -2.1 | 2.5 | 0.3 | 2.8 | -1.6 | 1.8 | 2 | 1.7 | 0.2 | 1 | -0.8 | 1 | 0.14 | 202 | 66.85 |
| 5 | -2.6 | 3.2 | -0.8 | 2.6 | -0.4 | 1.9 | 0.6 | 2.2 | -0.8 | 1.3 | -2.6 | 1.4 | -1.2 | 0.8 | 0.3 | 0.8 | 0.11 | 202 | 66.56 |
| 6 | -4.9 | 3.1 | -1.4 | 2.4 | -0.8 | 1.7 | -0.9 | 2.4 | -1.9 | 1.2 | -2.5 | 1.4 | -1.3 | 0.8 | 0.9 | 0.8 | 0.14 | 202 | 44.77 |
| 7 | -4.3 | 3.9 | -6.2 | 3.2 | -4.3 | 2.6 | -4.7 | 2.9 | -4.9 | 1.6 | 1.1 | 1.8 | -1 | 1.1 | 1.4 | 1.1 | 0.34 | 202 | 24.14 |
| 8 | -4 | 4.9 | -8.5 | 5.1 | -5.3 | 4.8 | -5.6 | 4.4 | -5.9 | 2.4 | 1.6 | 2.9 | -1.9 | 1.6 | 2.3 | 1.6 | 0.6 | 202 | 10.36 |
| 9 | -2.5 | 3.1 | -6.8 | 3.6 | -4.2 | 3.2 | -3.4 | 2.7 | -4.1 | 1.6 | 2.1 | 1.9 | -0.9 | 1 | 1.1 | 1 | 0.52 | 202 | 3.35 |
| 10 | -1.7 | 2.5 | -4.5 | 2.3 | -2.8 | 1.4 | -1.3 | 1.5 | -2.5 | 1 | 2 | 1 | -0.1 | 0.6 | -0.2 | 0.6 | 0.22 | 202 | 1.48 |
| 11 | 1 | 4.8 | -5.4 | 4.6 | -3 | 3.5 | -0.4 | 3.9 | -2.1 | 2.2 | 3.8 | 2.1 | 0.8 | 1.2 | -1 | 1.2 | 0.18 | 202 | 66.02 |
| 12 | 0.3 | 4.6 | -4.9 | 4 | -2.6 | 2.9 | 0 | 3.2 | -1.9 | 2 | 3 | 1.9 | 0.6 | 1.1 | -1 | 1.1 | 0.18 | 202 | 132.96 |
| 13 | 0.3 | 4.6 | -4.8 | 3.9 | -2.5 | 2.8 | 0 | 3.1 | -1.8 | 1.9 | 2.8 | 1.9 | 0.5 | 1.1 | -0.9 | 1.1 | 0.17 | 202 | 108.39 |
| 14 | -3.3 | 4 | -7.7 | 4.3 | -4.9 | 4.1 | -4.7 | 3.6 | -5.1 | 2 | 1.7 | 2.4 | -1.6 | 1.3 | 1.9 | 1.3 | 0.59 | 202 | 15.19 |
| 15 | -1.5 | 2.8 | -3.9 | 2.3 | -2.1 | 1.4 | -0.7 | 1.7 | -2.1 | 1.1 | 1.3 | 1 | -0.2 | 0.6 | -0.2 | 0.6 | 0.18 | 202 | 308.13 |
| 16 | -1.4 | 2.8 | -3.8 | 2.3 | -2.2 | 1.4 | -0.7 | 1.7 | -2.1 | 1.1 | 1.6 | 1.1 | 0 | 0.6 | -0.3 | 0.6 | 0.18 | 202 | 308.19 |

|  |  |  |  |  |  |  |  | Belsk - Table 10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| level | DJF | e_djf | MAM | e_mam | JJA | e_jja | SON | e_son | year | e_year | a | err_a | b | err_b | c | err_c | parameter | N | mmean |
| 1 | -9.3 | 12.2 | -7.8 | 4.7 | -3.5 | 4.1 | -0.2 | 4.6 | -5.3 | 3.3 | 4.6 | 3.7 | 0 | 1.8 | -0.6 | 1.6 | -0.08 | 134 | 35.17 |
| 2 | -13.4 | 15.8 | -9.6 | 5.2 | -5.3 | 5 | -2.4 | 5.9 | -8.5 | 4.8 | 3.3 | 3.6 | 0.5 | 1.8 | -0.9 | 1.6 | -0.06 | 134 | 36.22 |
| 3 | -9.3 | 15.4 | -7.9 | 5.2 | -4.8 | 5.1 | -2.6 | 6 | -6.6 | 4.9 | 3.4 | 4.1 | 0.8 | 2 | -1.1 | 1.9 | -0.03 | 134 | 55.87 |
| 4 | -4.1 | 12.8 | -4 | 4.2 | -3.7 | 3.8 | -0.8 | 4.6 | -3.3 | 3.8 | 2.1 | 3.2 | -0.4 | 1.5 | -1 | 1.5 | 0.03 | 134 | 71.13 |
| 5 | 0.1 | 8.8 | -0.3 | 3 | -0.6 | 2.8 | 0.6 | 3.5 | -0.1 | 2.6 | 1.9 | 2.4 | -1.4 | 1.1 | -0.1 | 1.1 | 0.01 | 134 | 57.67 |
| 6 | -1.8 | 8.3 | -1.8 | 3.1 | -0.9 | 3.2 | 2.5 | 4.9 | -0.6 | 2.5 | 1.9 | 2.8 | -2.2 | 1.4 | 0.5 | 1.4 | 0.01 | 134 | 36.63 |
| 7 | -3.8 | 6.9 | -3.8 | 3.4 | -2.2 | 3.4 | -0.2 | 3.7 | -2.5 | 2.2 | 0.8 | 2.7 | -1 | 1.4 | -0.6 | 1.4 | -0.17 | 134 | 21.16 |
| 8 | -1.7 | 5.3 | -4.8 | 3.7 | -2.1 | 4 | -3.1 | 3.8 | -3 | 2.1 | 1 | 2.9 | -0.1 | 1.4 | -1.6 | 1.4 | -0.18 | 134 | 9.83 |
| 9 | -2.9 | 5.4 | -5.6 | 2.8 | -1.9 | 3.1 | -3.5 | 2.6 | -3.5 | 1.9 | -0.1 | 2.1 | 0 | 1 | -1.4 | 1 | -0.1 | 134 | 3.38 |
| 10 | -4.4 | 7.8 | -4.9 | 2.5 | -1.3 | 2.5 | -1.5 | 2.8 | -3.1 | 2.4 | 1.5 | 2 | 0.1 | 0.9 | -1.4 | 0.9 | -0.04 | 134 | 1.51 |
| 11 | -10.7 | 15.5 | -8.6 | 5.3 | -5 | 5.1 | -2.5 | 6 | -7.3 | 4.8 | 3.4 | 4 | 0.7 | 2 | -1.1 | 1.8 | -0.04 | 134 | 92.13 |
| 12 | -7.8 | 14 | -6.8 | 4.7 | -4.4 | 4.4 | -1.6 | 5.3 | -5.5 | 4.3 | 3.2 | 3.6 | 0.3 | 1.8 | -1.1 | 1.6 | -0.01 | 134 | 163.19 |
| 13 | -6.5 | 13.7 | -5.8 | 4.5 | -4.2 | 4.2 | -1.5 | 5.2 | -4.7 | 4.2 | 2.9 | 3.5 | 0.1 | 1.7 | -1.1 | 1.6 | 0 | 134 | 126.95 |
| 14 | -2.3 | 4.8 | -5.3 | 3.2 | -2.2 | 3.5 | -3.4 | 3 | -3.3 | 1.9 | 0.5 | 2.5 | -0.1 | 1.2 | -1.4 | 1.2 | -0.14 | 134 | 14.74 |
| 15 | -5.5 | 9.8 | -5.1 | 2.9 | -3 | 2.5 | -0.7 | 3.6 | -3.8 | 2.8 | 2.5 | 2.3 | -0.1 | 1.1 | -1.2 | 1.1 | 0.03 | 134 | 329.36 |
| 16 | -6.3 | 9.6 | -4.9 | 3 | -1.8 | 2.7 | -0.9 | 3.4 | -3.6 | 2.7 | 2.6 | 2.4 | -0.2 | 1.1 | -1.1 | 1.1 | 0.02 | 134 | 333.21 |

|  |  |  |  |  |  |  |  | Arosa | Table 11 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| level | DJF | e_djf | MAM | e_mam | JJA | e_jja | SON | e_son | year | e year | a | err_a | b | err_b | c | err_c | parameter | N | mmean |
| 1 | -5.3 | 3.9 | -8.7 | 3.9 | -3.9 | 2.7 | -3.1 | 3.6 | -5.4 | 1.8 | 0.1 | 2 | -0.4 | 1.2 | 0.1 | 1.2 | 0.13 | 209 | 28.92 |
| 2 | -4 | 3.5 | -8.5 | 3.6 | -2.1 | 2.8 | -1.7 | 3 | -4.9 | 1.8 | 0.3 | 1.5 | 0.2 | 0.9 | -0.3 | 0.8 | 0 | 209 | 29.71 |
| 3 | -3.6 | 4 | -7 | 3.8 | -1.6 | 2.9 | -0.4 | 2.9 | -3.6 | 1.8 | 0.5 | 1.8 | 0.1 | 1 | -0.4 | 1 | 0.03 | 209 | 49.59 |
| 4 | -0.4 | 3.2 | -3.3 | 3.1 | 0.9 | 2.3 | 1.9 | 2.4 | -0.4 | 1.4 | 1.5 | 1.5 | -0.4 | 0.9 | -0.1 | 0.9 | 0.02 | 209 | 73.05 |
| 5 | 2.5 | 2.9 | 0.3 | 2.9 | 2 | 2.1 | 2.5 | 2.2 | 1.8 | 1.3 | 1.8 | 1.5 | -0.9 | 0.8 | 0.4 | 0.8 | 0.18 | 209 | 61.98 |
| 6 | -1.4 | 3.2 | -0.9 | 2.9 | -1.7 | 2 | -1.4 | 2.4 | -1.3 | 1.3 | 1.4 | 1.5 | -0.6 | 0.9 | -0.2 | 0.9 | 0.24 | 209 | 39.52 |
| 7 | -6.9 | 2.7 | -5.3 | 3 | -5 | 2.2 | -4.8 | 2.5 | -5.5 | 1.3 | 1.2 | 1.5 | -0.2 | 0.9 | -0.3 | 0.9 | 0.19 | 209 | 22.52 |
| 8 | -7.2 | 3.4 | -6.7 | 4.3 | -5.1 | 3.5 | -4.4 | 3.4 | -5.9 | 1.8 | 1.2 | 2.2 | -0.3 | 1.3 | 0.3 | 1.3 | 0.32 | 209 | 10.35 |
| 9 | -5.6 | 2.5 | -5.6 | 3.5 | -3.1 | 2.7 | -2.5 | 2.4 | -4.3 | 1.4 | 1.1 | 1.6 | -0.2 | 0.9 | 0.2 | 0.9 | 0.27 | 209 | 3.46 |
| 10 | -3.8 | 2 | -4.6 | 2.6 | -1.9 | 1.6 | -0.6 | 1.6 | -2.8 | 1 | 1.2 | 1.1 | -0.2 | 0.6 | -0.1 | 0.6 | 0.12 | 209 | 1.54 |
| 11 | -3.7 | 3.7 | -7.6 | 3.7 | -1.8 | 2.8 | -0.8 | 3 | -4.1 | 1.8 | 0.4 | 1.7 | 0.2 | 1 | -0.4 | 0.9 | 0.02 | 209 | 79.31 |
| 12 | -2.2 | 3.4 | -5.8 | 3.3 | -0.5 | 2.5 | 0.6 | 2.6 | -2.4 | 1.6 | 0.9 | 1.6 | -0.1 | 0.9 | -0.3 | 0.9 | 0.01 | 209 | 152.45 |
| 13 | -1.8 | 3.4 | -4.9 | 3.3 | -0.1 | 2.4 | 1 | 2.5 | -1.7 | 1.5 | 1.1 | 1.6 | -0.2 | 0.9 | -0.2 | 0.9 | 0.02 | 209 | 122.7 |
| 14 | -6.4 | 2.9 | -6.3 | 3.9 | -4.4 | 3.1 | -3.6 | 2.9 | -5.2 | 1.6 | 1.1 | 1.9 | -0.3 | 1.1 | 0.2 | 1.1 | 0.31 | 209 | 15.35 |
| 15 | -2 | 2.2 | -4.3 | 2.4 | -0.9 | 1.6 | 0 | 1.8 | -1.9 | 1 | 1.3 | 1.1 | -0.2 | 0.7 | -0.2 | 0.6 | 0.07 | 209 | 320.12 |
| 16 | -2.8 | 2.1 | -4.4 | 2.4 | -1.1 | 1.5 | 0.1 | 1.8 | -2.2 | 1 | 1.3 | 1.1 | -0.3 | 0.6 | -0.2 | 0.6 | 0.06 | 209 | 322.59 |

|  |  |  |  |  |  |  |  | OHP | Table 12 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| level | DJF | e_djf | MAM | e_mam | JJA | e_jja | SON e | _son | year | e_year | a | err_a | b | err_b | c | err_c | Param. | N | mmean |
| 1 | -2.3 | 8.3 | -6.6 | 6.8 | 1.8 | 6.6 | 0.1 | 7.7 | -1.8 | 3.6 | 1.1 | 3.3 | -1.8 | 2.1 | 1.4 | 2.1 | 0.28 | 151 | 33.12 |
| 2 | -2.9 | 6.7 | -8.8 | 6.7 | -0.6 | 6 | 1 | 7.4 | -4 | 3.5 | 1.2 | 2.7 | -0.9 | 1.7 | 1 | 1.6 | 0.17 | 151 | 28.76 |
| 3 | -2.7 | 7.1 | -9 | 6.3 | -2.5 | 5.3 | 0.1 | 6 | -4 | 3.2 | 1.4 | 2.6 | -1 | 1.7 | 0.9 | 1.6 | 0.15 | 151 | 48.48 |
| 4 | -2 | 5.5 | -6.6 | 4.2 | -3.7 | 3.2 | -0.6 | 4.3 | -3.4 | 2.2 | 1.6 | 1.8 | -0.8 | 1.1 | 0.5 | 1.1 | 0.11 | 151 | 74.68 |
| 5 | -1.1 | 3.6 | -3.2 | 3.2 | -2.4 | 2.1 | -0.7 | 2.6 | -1.9 | 1.5 | 0.5 | 1.3 | -0.1 | 0.8 | -0.1 | 0.8 | 0.22 | 151 | 66.77 |
| 6 | -2.3 | 4.1 | -2.4 | 3.2 | -0.9 | 2.3 | -1.8 | 3 | -1.8 | 1.6 | -0.3 | 1.4 | 0.2 | 0.8 | -0.5 | 0.9 | 0.19 | 151 | 42.42 |
| 7 | -7.5 | 4 | -7 | 3.5 | -2.1 | 3.7 | -5.4 | 2.7 | -5.5 | 1.8 | -1.5 | 1.7 | -0.1 | 1 | -0.1 | 0.9 | 0.13 | 151 | 23.38 |
| 8 | -7.7 | 3.7 | -7.6 | 2.5 | -1.9 | 3.9 | -7.1 | 2.4 | -6.2 | 1.6 | -1.9 | 1.4 | -1 | 0.8 | 1 | 0.8 | 0.04 | 151 | 10.3 |
| 9 | -4.9 | 2.9 | -6.4 | 2.7 | -1.2 | 3.2 | -4.2 | 1.9 | -4.3 | 1.3 | -0.5 | 1.2 | -1.1 | 0.7 | 1 | 0.7 | 0.06 | 151 | 3.42 |
| 10 | -2.9 | 3.3 | -6.4 | 3.4 | -1.4 | 2.9 | -1.4 | 2.4 | -3.1 | 1.5 | 0.4 | 1.4 | -0.9 | 0.8 | 0.4 | 0.8 | 0.12 | 151 | 1.55 |
| 11 | -2.6 | 7 | -8.9 | 6.4 | -1.7 | 5.6 | 0.4 | 6.5 | -4 | 3.3 | 1.4 | 2.7 | -1 | 1.7 | 1 | 1.6 | 0.16 | 151 | 77.15 |
| 12 | -2.3 | 6.2 | -8 | 5.3 | -2.8 | 4.3 | -0.1 | 5.2 | -3.7 | 2.7 | 1.5 | 2.3 | -0.9 | 1.4 | 0.7 | 1.4 | 0.14 | 151 | 151.92 |
| 13 | -2.3 | 6.1 | -7.7 | 4.9 | -3.3 | 3.9 | -0.4 | 4.8 | -3.7 | 2.6 | 1.6 | 2.1 | -0.9 | 1.3 | 0.7 | 1.3 | 0.12 | 151 | 123.26 |
| 14 | -6.6 | 3 | -7.2 | 2.3 | -1.7 | 3.4 | -6.1 | 2 | -5.5 | 1.4 | -1.2 | 1.3 | -1.1 | 0.7 | 1 | 0.7 | 0.03 | 151 | 15.29 |
| 15 | -2.3 | 3.9 | -5.9 | 3.7 | -1.8 | 3.1 | -1.2 | 3.5 | -3 | 1.8 | 0.8 | 1.7 | -0.7 | 1 | 0.4 | 1 | 0.21 | 151 | 332.38 |
| 16 | -2.4 | 4.1 | -6 | 3.7 | -1.5 | 3.1 | -0.6 | 3.5 | -2.8 | 1.8 | 0.8 | 1.7 | -0.8 | 1 | 0.4 | 1 | 0.2 | 151 | 332.97 |

All trend values are in % per decade, the error is mean 2 sigma, "a" is the amplitude of the solar cycle in % (from 120), b and c are the amplitudes of QBO for wind at 30 and 50 mb respectively. The last column shows the mean ozone in DU at the beginning of the trend calculation period (1978).

Rows 11 is (layers 2 and 3) together, row 12 is (layers 2 to 4), row 13 is (layers 3 and 4) together, row 14 is (layers 8 and 9) together, and rows 15, 16 are the total ozone observed and retrieved by Umkehr correspondingly.

All these trend models are based on estimation of monthly trends, i.e. we use 12 linear functions in the model. The "seasonal" model tends to smooth everything, for example for Arosa it makes winter-spring trend less negative but it also reduces positive trends in fall.

## Trend Results

Figure 1t (Part D, Trends) (B+Ar+OHP period 1978-97 and 1978-99) show that the major ozone decline is observed in the lower and the upper stratosphere. This fact is confirmed also by data from ozonesondes (e.g. Bojkov and Fioletov, 1995) and from satellites (WMO Ozone Report #43 and the references therein). This Figure also illustrates the effect of adding two more years of data when it is known that 1998 and 1999 were years with relatively higher total ozone than most of the previous years. It is obvious that adding the extra two years have significantly changed (reduced by nearly one half) the negative trend rates in the lower stratosphere while non-significantly changed the rates in the upper stratosphere. A similar result is achieved by adding these two higher ozone years in the end of even longer record 1964-97 compared with 1964-99 for the combined plot of Belsk and Arosa.

Now the question of what is the effect of one intervention in the Belsk record on the combined trends over Central and Western Europe. The two Figures of (B+AR+OHP) for the 78-99 and 78-97 periods show that the Belsk intervention did not cause statistically significant change in the European overall (combined) trends. It is noted in the case 1978-99 that the intervention caused the trends in the lower stratosphere to be slightly (up to 0.5%per decade in layer IV) more negative, and caused some similarly small move toward less negative trends in the upper layers (largest effect about 0.4% in layer VIII). The effect on the 1978-97 period is even smaller.

The last three figures on (Part D, trends) are Sapporo with no intervention and with only one at 6/89 applied until the end of observing period (e.g.1999). The two derived trends profiles are completely reversed like a reverse mirror image, which could not be true. Therefore, only one correction of intervention in 6/89 is not enough. The second Figure (TD10) shows how successfully this problem could be solved if one uses simultaneous corrections of all identified important interventions when calculating trends.

The illustration of the different effect of each intervention is seen in the figure TD11 for NAHA where the black line shows how the trends behave if one uses an intervention correction applied on 1/89 for the period 78-97. This important intervention completely reverses the trend profile towards a more reasonable estimation comparable to other stations in the tropical region.

We also present a few of the seasonal trends for selected combination of stations for Europe. These are figures in part E. In these figures we see that the decline in the layers VII - X is significant -5 to -7% (-3% in SON,), while in layer V the trends are all not significant. The max decline is during the two seasons (DJF and MAM) when also below layer V the decline is  -5 to -9%, and in SON the decline below layer VII are NOT significant.

The 3 natural seasons are in concurrence with the 4 seasons and somewhat better reflecting the overall annual trend. The decline in layers VII to X are by  -4 to -6%, and layer V trends are NOT significant, while max decline is in DJFM when below layer V is  -5 to -8%. SON is of course identical with the 4 seasons. In all these calculations the terminology SON, DJF, MAM, JJA is referring to the seasonal separation of the data (as explained in the beginning of the report), thus in the trend analysis for SON, for example, only data from the fall season have been used.

Finally in part F we present figures (TF1 till TF 6) for the solar cycle component and the QBO component in selected combinations of stations. The error bars for the trends and other parameters seem to be too small for the 1964-99 period.  It is likely due to some underestimation of the autocorrelation.  The measurements were less frequent (and therefore monthly averages were less auto correlated) in the 1960’s than in more recent years when the declining ozone trend actually occurred.  The autocorrelation, however, was estimated for the entire period.

The trend in layer VII at Arosa is very strong with very small error. Such trend should be seen from the data without any complicated calculations.  It looks the data do show a strong decline with small variability at that level.

The amplitude of the solar cycle estimations for individual stations is only 1-2% and it can be easily masked by small errors in instrument calibrations.  All stations show some solar cycle-related components in their total ozone records with amplitudes from 0 to 2.5%. These are similar to the magnitudes referred to for example by Bojkov and Fioletov (1995), and in WMO Ozone Assessments 1994 and 1998.  However, it is hard to find clear latitudinal or altitudinal dependence in the solar cycle-related component due to large errors and relatively short period of observations.

The QBO-related component is not properly estimated in the trend statistical model.  The statistical model does not take into account seasonal dependence of the QBO-related signal in extra equatorial ozone. It is done because seasonal dependence of QBO has practically no effect on the trend, but require extra parameters in the statistical model.  Also the weighting procedure gives higher weights to summer and fall months when the ozone variability is low and when the QBO signal is the smallest. Two predictors (wind at 30 and 50 mb) are used in the statistical model to describe the QBO related signal.  The amplitude of the QBO-related signal can be written as

sqrt<( (b*W30)**2 + (c*W50)**2)>

Where <> means average, r is the correlation coefficient between W30 and W50, b**2=b*b and the actual value of r is 0.58.

As mentioned above, the variations of the vertical distribution of ozone as depicted in the Umkehr layers can be described by a statistical model of the form:

Ozt  = (Annual cycle) + (trend) + (Solar effect) + (QBO effect) + Noise

Where Ozt represents the monthly mean ozone values at a specific Umkehr layer.

As an example of the terms of QBO, we present here the effects of QBO and the solar cycle at the station of Tateno (36N) in the following figure.

The average QBO component over the whole record is presented here, as percent of the annual mean, and with respect to the transition month from the east to west phase, accounted for as lag 0 (key-dates).

The peak to trough range of the QBO component varies from about 2 to 6%, depending on the layer, highest at altitudes from 18 – 25 km.

The phase lag at the level of maximum effect in the lower stratosphere is about 8 months, as expected from similar studies on total ozone (e.g.Zerefos et al., 1992, 1994), with a distinct phase progression of about 1-2 kms / month from higher to lower altitudes.

Effect of the QBO in the Umkehr layers using the relationship with the 30 and 50hPa equatorial wind series.

If we use the above results to derive the integrated change of ozone from the individual trend of each layer and compare that with the change (in m-atm-cm) from the total ozone trend, we can derive very useful information about the quality of the derived Umkehr profiles. In the following table the integrated change from each layer individually can be seen compared with the change from the total ozone for two periods. These periods are from 1978 to 1997 (20 years) and 1964 to 1999 (35 years) and they are the periods used throughout the REVUE analysis. In table 13 one can see that in stations with good quality Umkehr data the integrated layers trends are almost identical to the total ozone changes estimated from the total ozone trends. Nevertheless in some stations even though there is relative agreement the difference is indicative that some part of the station’s record may be not of the best quality. For example Cairo, and New Delhi although their records are long, show some discrepancies, possibly sometimes two or three years with no data of good quality (using the criteria illustrated in this report) are distorting the trends. Kagoshima has obvious problems with missing periods along with the fact that a volcano exists in close distance to the station, which also distorts the data giving more spread and bad quality data in some periods. It should be kept in mind also that practically at all sub-tropical stations the total ozone trends as well as most of the individual layers trends are statistically not significant and therefore one should not be surprised that there are differences between the integrated ozone amounts.

| ###### TABLE 13 | 78-97 (matm-cm) |  | 64-99 (matm-cm) |  |
| --- | --- | --- | --- | --- |
|  | Integrated change form layers trends | Integrated change form Total O3 | Integrated change form layers trends | Integrated change form Total O3 |
| Boulder | -12.9 | -13.0 |  |  |
| Belsk | -24.5 | -25.0 | -11.8 | -11.5 |
| Arosa | -13.1 | -12.2 | -15.9 | -15.8 |
| OHP | -20.8 | -19.9 |  |  |
| Cairo | -2.1 | -4.1 |  |  |
| Aswan | -11.7 | -11.8 |  |  |
| New Delhi | -2.8 | 0.0 | -9.2 | -7.8 |
| Sapporo | -18.6 | -22.5 | -22.0 | -22.7 |
| Tateno | -10.9 | -10.8 | -12.7 | -12.3 |
| Kagoshima | -0.2 | 4.5 | -5.9 | -3.0 |
| Naha | -15.7 | -8.6 |  |  |
| Mauna Loa | -4.1 | -4.2 |  |  |
| Perth | -9.7 | -12.3 | -6.4 | -9.1 |

Shadowed are the numbers from stations that are considered of bad quality.

Finally we present in the following two figures, for reference, a comparison with the trend results from the assessment of WMO-SPARC/IO3C/GAW (figure 3.14) for the vertical distribution of ozone. The large circles and squares are the values for the combined layers 2+3+4 (bottom) and 8+9+10 (top). Also we present a comparison with the old (1992 ) and the new algorithm in conjunction with recent studies in the literature. (Reinsel et. al.)

Also in the next table we provide trends and their errors for 4 latitude zones.

| Period78-97 | 2+3+4 | Err. | 3+4 | Err. | 4 | Err. | 5 | Err. | 6 | Err. | 7 | Err. | 7+8+10 | Err. | Total O3 | Err. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19-24 deg. | 3.1 | 3.0 | 3.0 | 3.2 | 3.0 | 3.5 | -0.9 | 4.2 | -3.8 | 2.5 | -7.0 | 4.4 | -6.3 | 3.2 | -0.3 | 1.6 |
| 26-30 deg. | 1.4 | 2.3 | 1.5 | 2.3 | 1.3 | 2.3 | -1.8 | 1.3 | -5.6 | 1.6 | -4.9 | 2.4 | -4.1 | 1.9 | -0.8 | 1.0 |
| 36-40 deg. | -2.1 | 1.8 | -2.1 | 1.8 | -2.1 | 1.8 | -1.5 | 1.0 | -1.3 | 1.4 | -3.1 | 1.8 | -3.9 | 1.5 | -1.9 | 1.1 |
| 44-52 deg. | -3.9 | 1.6 | -3.4 | 1.5 | -2.4 | 1.4 | -0.1 | 1.3 | -1.2 | 1.3 | -4.5 | 1.3 | -4.7 | 1.6 | -2.9 | 1.0 |

# V.REFERENCES

Bass, A.M. and Paur, R.J., Proc. Quadrennial Ozone Symp. Chalkidiki, Greece eds. C. Zerefos and A. Ghazi. Reidel, Dordrecht, 1984, 606p.

Bojkov, R., W. D. Komhyre, A. Lapworth and K, Vanicek, Handbook for Dobson Ozone Data Re-Evaluation, Global Ozone Research Project, WMO Report No. 29, ed. R. D. Hudson and W. G. Planet,  1993.

Bojkov R.D., .Bishop and V.Fioletov. Total ozone trends from quality-controlled ground-based data 1964-1994 in JGR, 100, p. 25,867-25,876, 1995)

Bojkov R.D. and V.Fioletov, Changes of the lower stratospheric ozone over Europe., JGR Vol 102, NO D1, 1337-1347, 1995

Bojkov, R.D., L. Bishop, W.J. Hill, G.C. Reinsel, G.C. Tiao, A statistical trend analysis of revised Dobson Total Ozone stations over the northern hemisphere. JGR, 95, 9785-9807, 1990

Bojkov Rumen D., Vertical ozone distribution characteristics deduced from 44,000 re-evaluated Umkehr Profiles from the 1957-1999 period, Intern. O3Symp, Sapporo, Japan, 2000

DeLuisi, J. J., D. U. Longenecker, P. K. Bhartia, S. Taylor, and C. L. Mateer, Ozone profiles by Umkehr, SBUV, and Ozonesonde: A Comparison including the inversion algorithms for Umkehr and SBUV, in Ozone in the Atmosphere, edited by R. D. Bojkov and P. Fabian, pp. 206-210, A Deepak, Hampton, Va., 1989a.

DeLuisi, J. J., D. U. Longenecker, C. L. Mateer, and D. J. Wuebbles, An analysis of northern mid-latitude Umkehr measurements corrected for stratospheric aerosols for 1979-1986, J. Geophys. Res., 94, pp. 9837-9846, 1989b.

DeLuisi, J. J., D. U. Longenecker, C. L. Mateer, and W.P. Chu, Estimation of solar backscatter Ultra-Violet albedo using ground-based Umkehr measurements, J. Geophys. Res., 98, pp. 9837-9846, 1993.

DeLuisi, J. J., C. L. Mateer, D. Theisen, P.K. Bhartia, D. Longenecker, and B. Chu, Northern mid-latitude ozone profile features and trends observed by SBUV and Umkehr, 1979-1990, J. Geophys. Res., 99, pp. 18,901-18,908, 1994.

DeLuisi, J.J.,  Personal communication - Presentation in the kick off meeting for REVUE, Paris, FRANCE 1996

DeLuisi, J.J., I.V. Petropavlovskikh, C.L. Mateer, An Evaluation of the Uncertainties in Estimating Stratospheric Aerosol Errors to Retrieved Umkehr Ozone Profiles, Proceedings from the XVIII Quadrennial Ozone Symposium in L’Aquila, Italy, ed. by R. Bojkov and G. Visconti, p.115-118, 1997.

Deluisi J.J., Aerosol Error Corrections in Umkehr Retrieved Ozone Profile Record, Intern. O3Symp, Sapporo, Japan, 2000

Godin S., Reconstruction of Vertical ozone distribution from Umkehr Estimates REVUE, Intern. O3Symp, Sapporo, Japan, 2000

Komhyr, W. D., Operations Handbook – Ozone Observations with a Dobson Spectrophotometer, Global Ozone Research Project, WMO Report No. 6, 1980.

Kosmidis, E.,  C. S. Zerefos, R. D. Bojkov, J. D. Deluisi, I. Petropavlovskikh and D. Thuissen, Re-Evaluation of Raw Umkehr data, Intern. O3Symp, Sapporo, Japan, 2000

Mateer, C.L. and J.J. DeLuisi, A new inversion algorithm, J.Atm. And Terr. Phys., 54, 537-556, 1992

Miller, A.J., L.E. Flynn, S.M. Hollandsworth, J.J. DeLuisi, I.V. Petropavlovskikh, G.C. Tiao, G.C. Reinsel, D. J. Wuebbles, J. Kerr, R.M. Nagatani, L. Bishop, and C.H. Jackman, Information content of Umkehr and solar back-scattered ultraviolet (SBUV) 2 satellite data for ozone trends and solar responses in the stratosphere, J. Geophys. Res., 102, D15, 19,257-19,263, 1997.

Newchurch, M. J., and D. M. Cunnold, Aerosol effect on Umkehr ozone profiles using Stratospheric Aerosol and Gas Experiment II measurements, J. Geophys. Res., 99,  1383-1388, 1994.

Newchurch, M. J., D. M. Cunnold, and J. Cao, Intercomparison of SAGE with Umkehr[64] and Umkehr[92] ozone profiles and time series: 1979-1991, J. Geophys. Res., 103,  31,277-31,292, 1998

Petropavlovskikh, I.V., B. Herman, R. Lougman, P. K. Bhartia, C. L. Mateer,  J. Lenoble, Y. Belikov, A Comparison of Radiance Calculations by Spherical Atmosphere Radiation Transfer Codes for Modeling the Umkehr Effect, Proceedings from the XVIII Quadrennial Ozone Symposium in L’Aquila, Italy, ed. by R. Bojkov and G. Visconti, p. 163-166, 1997.

Petropavlovskikh, I.V., L. Flynn, J. DeLuisi, P.K. Bhartia, A Critical Look at the Capabilities of the Umkehr and SBUV for Ozone Information Below 20 km, Proceedings from the XVIII Quadrennial Ozone Symposium in L’Aquila, Italy, Ed. by R. Bojkov and G. Visconti, p. 159-162, 1997.

Petropavlovskikh, I. V., DeLuisi, J. J. , Loughman,  R. , A Comparison ofUV  Intensities 	Calculated by Spherical-Atmosphere Radiation Transfer Codes:  Implications for Aerosol Corrections to the Umkehr Method, (in press, JGR,  2000)

Petropavlovskikh, I. V., J. DeLuisi, D. Theisen, L. Flynn, B. Chu, Investigation of Systematic and RMS Differences Among Ozone Profiles Observed by the Umkehr, SBUV, and SAGE, in review, JGR, 1999b

Petropavlovskikh, I.V., John J. DeLuisi, David Theisen, Rumen D. Bojkov and E. Kosmidis. "Detection of Shifts in the Umkehr N-value Radiance Records" by (1999, GRL, in press).

Petropavlovskikh I., Detection of Shifts in the Umkehr N-value Radiance Records, Intern. O3Symp, Sapporo, Japan, 2000

Petropavlovskikh I., Umkehr Retrieval Algorithm: Assessment of Uncertainties, Intern. O3Symp, Sapporo, Japan, 2000

Reinsel, G. C., G. C. Tiao, J. J. DeLuisi, C. L. Mateer, A. J. Miller, and J. E. Frederick, Analyses of upper stratospheric Umkehr ozone profile data for trends and the effects of stratospheric aerosols, J. Geophys. Res., 89, 4833-4840, 1984.

Reinsel, G. C., G. C. Tiao, J. J. DeLuisi, S. Basu, and K. Carriere, Trend analyses of aerosol-corrected Umkehr ozone profile data through 1987, J. Geophys. Res., 94, 16,373-16,386, 1989.

Reinsel, G. C., W.-K. Tam, and L. H. Ying, Comparison of trend analyses for Umkehr data using new and previous inversion algorithm, Geophys. Res. Lett., 21, 1007-1010, 1994.

Reinsel, G. C., G. C. Tiao, A.J. Miller, R.M. Nagatani, E.C. Weatherhead, S. D.J. Wuebbles, W.-K. Cheang, L.Zhang, L.E.Flynn, and J.B.Kerr. Update of Umkehr ozone profile data trend analysis through 1987, J. Geophys. Res., 104, No D19, 23,881-23,898, 1999.

Stevermer, A., I. Petropavlovskikh, J. Rosen, F. Congeduti, J. Deluisi, W. Chu, Conversion of SAGE II, lidar, and broadband turbidity measurements to aerosol extinction at UV wavelengths, Proceedings, 1998 AGU Fall Meeting, December 6-10, San Francisco, CA F171:A42A-17, (1998).

Stevermer A. J, I. V. Petropavlovskikh, J. J. DeLuisi, J. M. Rosen, “Development of a Global Stratospheric Aerosol Climatology: Optical Properties and Applications for UV “, (in review, JGR, 2000)

Weatherhead, E.C., G.C. Tiao, G.C. Reinsel, J.E. Frederick, J.J. DeLuisi, D.C. Choi, and W.Tam, Analysis of long-term behavior of ultraviolet radiation measured by Robertson- Berger meters at 14 sites in the United States, J. of Geophys. Res., Vol. 102, No. D7, April 20, 1997, pp. 8737-8754, 1997.

WMO Ozone Assessment - 1998 (WMO Ozone Report # 44)

WMO/IO3C/SPARC Assessment of Trends in the Vertical Distribution of Ozone, WMO Ozone Project Report #43, Geneva 1998

Figures 1-15: Time series of Monthly Values for Each station as they are included in the analysis.

Some Sample Aerosol Corrections.

Figure 20-24: New comparisons with O3Sondes using the 99 algorithm.

PART B : INDIVIDUAL REPORT FROM SA-CNRS

Contractor : Service d'Aéronomie CNRS

Leading Scientists : Dr. Sophie Godin

Address :	Université Pierre et Marie Curie

Service d'Aéronomie du CNRS Boite 102

4 Place Jussieu

75230 Paris Cedex 05

Telephone :	33 1 44274767

Fax : 		33 1 44273776

E-Mail : sophie.godin@aero.jussieu.fr

Reporting period: March 1998 – December 1999

I. Objectives for the reporting period

The objectives of the reporting period concern essentially the validation of the new Umkehr algorithm: The following studies were performed:

- Validation of the re-evaluated Umkehr profiles by comparison with the multi-instrumental ozone record at OHP: stratospheric ozone lidar, ozone soundings, SAGE II measurements.

- Validation of the re-evaluated Umkehr profiles by comparison with the SAGE II satellite data at all the selected Umkehr stations.

II. Main results

- Comparison at OHP

The new Umkehr data provided by the LAP are compared extensively to the long-term record of stratospheric ozone vertical distribution at OHP. In addition with the Dobson-Umkehr data, measurements of the ozone vertical distribution are provided by ozone soundings since 1984 and by lidar since 1985. Regular ozone soundings began in 1984, initially to validate the ozone lidar measurements performed with dye lasers at that time. Brewer Mast sondes were used up to 1991, followed afterwards by ECC sondes which are the most common sondes used at the present time. First measurements of the stratospheric ozone profile with an excimer laser were obtained in July 1985 at OHP. Regular stratospheric ozone lidar measurements began at the end of 1986. In this study, we include also the SAGE II measurements obtained in the vicinity of OHP (within  5° in latitude and  10° in longitude), in order to compare the 3 types of ground-based measurements used for the long-term monitoring of the ozone vertical distribution with the longest satellite record. The ozone record considered in this study comprises the following measurements:

- 2705 umkehr profiles from September 1983 to December 1998.

- 798 SAGE profiles from October 1984 to April 1999.

- 1164 lidar profiles from July 1985 to December 1999.

- 376 sondes profiles (Brewer Mast and ECC) from October 1984 to December 1998.

The number of ozone profiles obtained each year at OHP from 1983 by the various instruments are represented in Figure 1a: This figure shows that Umkehr and SAGE provide an average of 170 and 50 measurements per year respectively. The Umkehr measurements obtained in 1992 were not included here, due to the perturbation of the Umkehr ozone measurements by the Pinatubo aerosols (see the LAP report on the influence of stratospheric aerosol on Umkehr ozone measurements). In 1996, a problem with the instrument reduced the number of profiles to 75.

Figure 1: (a) Number of ozone profiles per year obtained by the Umkehr, lidar, sondes and SAGE II instruments. (b) Average number of measurements per month over the whole period.

The number of measurements performed by the lidar increased continuously from 1985 to 1990 with the improvement of the operationality of the instrument. An average of 150 lidar measurements per year is obtained presently on this site, with a peak of 194 in 1997. The altitude range of the lidar ozone measurements has evolved since 1985: the minimum altitude was around 20 km up to 1991. It reached 30 km at several occasions after the eruption of Mount Pinatubo due to the presence of the aerosol volcanic cloud in the low stratosphere and decreased to the previous levels due to the decay of the volcanic cloud. Since April 1994, the minimum altitude is around 12 km thanks to the implementation of “raman” channels, which allow a better reliability of the lidar ozone measurement in the low stratosphere and are less sensitive to stratospheric aerosols. The lidar experimental set-up was completely modified at the end of 1993: in the new configuration, 3 ozone profiles are obtained simultaneously: 2 "rayleigh" profiles corresponding to the atmospheric elastic backscattering and 1 "raman" profile related to the first stokes vibrational raman scattering by molecular Nitrogen. The “rayleigh” signals provide an ozone profile from 16 km to 50 km, while the “raman” signals, less perturbed by volcanic aerosols, are used to test the linearity of the rayleigh signal in the low stratosphere and to provide an ozone profile from the tropopause to 25 - 30 km. A composite ozone profile is calculated from the “raman” and the “rayleigh” profiles, the “rayleigh” signals being used from the altitude where no saturation of the detectors and counting systems is detected.

The number of ozonesondes measurements at OHP is much lower than for the other instruments: due to the cost of these measurements only 4 measurements per month are performed presently since 1994 but an average of only 10 to 20 stratospheric measurements per year was obtained in the previous ten year period.

The monthly sampling of the various instruments is represented in Figure 1b. It shows that while the Umkehr, lidar and sonde provide a rather homogeneous sampling of the ozone field all year round, the SAGE measurements sample preferably the winter season (only 1 measurement in average in August), which questions the ability of this instrument to detect valuable trends in summer on this site, in spite of the lower ozone variability during this season. The monthly sampling of the ozone soundings and the lidar increased since 1993, with an average of respectively 4 and 10 to 15 measurements per month, at the present time.

The Umkehr measurements provide ozone data in 10 Umkehr layers located between the ground and 50 km, while the lidar, sondes and SAGE II measurements provide ozone measurements with a higher vertical resolution. For the comparison, the Umkehr data were provided in Dobson Units in each Umkehr layer and the other measurements (which measure ozone in partial pressure for the sondes and number density for the lidar and SAGE II) had to be converted to the same quantity. The lidar, sondes and SAGE II measurements were averaged on Umkehr layers which constitute the vertical scale of the Umkehr measurements and are characterised by a width of about 5 km. Note that while this value is the upper limit of the lidar resolution at 45 km, the resolution of the Umkehr measurements is much worse with a vertical resolution evaluated to 12 km in the International Ozone trend panel (1988). In order to integrate the SAGE II values in the Umkehr layers, the NCEP data provided with the SAGE II record were used. For the lidar, composite pressure-temperature profiles derived from radiosondes and temperature lidar measurements obtained on the same site were used. For the sondes, we used the pressure-temperature measurements made during the flight.

The comparisons between the Umkehr and the other data sets was performed for coincident measurements, within  24 hours. The number of coincidences is shown in figure 2 as a function of Umkehr layers. The variation of the number of coincidences with height is due to the temporal evolution of the altitude range of the ancillary measurements. In the the case of the lidar, this evolution is due to the change of the experimental set up and the perturbation of the measurements in the lower stratosphere by the Mount Pinatubo volcanic cloud. Similarly to other measurement based on optical techniques, the SAGE II measurements were also perturbed by the volcanic aerosol in the lower stratosphere. For this study, the data were filtered according to the recommendation of the WMO/SPARC ozone assessment (WMO, 1998). For the sondes, the evolution is due to the variation of the balloon burst altitude. From 1984 to 1999, more than 150 coincidences are found with the sondes, 300 with SAGE II and more than 400 for the lidar above layer 3. In the lidar case, the number of coincidences is heavily weigted towards the last years were more measurements were performed.

For all the measurements, the daily difference with the Umkehr data is defined as:

Dj=Uj−XjXj

where Xj is the sonde, lidar or SAGE II measurement on day j and Uj the Umkehr measurement on the same day within  24 hours. The comparison was performed for both the old and the new Umkehr retrieval. The old Umkehr retrieval corresponds to the data taken from WODC (raw data) without any corrections as explained in the LAP report, and using the old version of the algorithm.

Figure 2 : Number of coincident measurements at OHP

The average difference between the Umkehr data and the ancillary measurements at OHP is represented in figure 3. The error bars are the 2 sigma standard error, which allow to test an eventual bias between the measurements: if the error bars cross the zero line at a particular altitude, then no bias is detected. The results in the lowermost layers (1-4) have to be considered with caution, since the effect of A Priori information (or First Guess profile) on the Umkehr retrieval is greatest in these layers (see the annex II for more information on the the sensitivity of the Umkehr algorithm to the First Guess). In the present retrieval, fixed A Priori information (FAP) was used in order to minimize the bias to the total trend caused by the use of total ozone as A Priori. However, FAP does not provide the best First Guess for the individual retrieval. Therefore, the comparisons between ancillary measurements and Umkehr are discussed only from layer 3, keeping in mind that the optimum range for the algorithm is from layer 5 to 9.

The comparison between the lidar data and both Umkehr retrievals shows very little difference except in layer 4. The new Umkehr algorithm provides lower ozone values in this layer, which changes the sign of the average difference. From layer 5 to 9 virtually no bias is seen between the lidar data and both retrievals. In layer 9, the comparison is based on 16 events only due to the loss of precision of the lidar in the upper stratosphere. The comparison with SAGE II looks very similar except in layer 9 where a significant positive bias is seen. As in the case of the lidar, the positive bias in layer 4 becomes negative with the new retrieval. In layer 3 the negative biais is larger in absolute value. The comparison with the sondes provides a somewhat different picture, with a positive biais in the whole altitude range. On can note that the comparison in layer 6 is based on 15 comparisons only, since the balloon hardly reach altitudes above 32 km necessary to provide ozone values in layer 6. No bias is seen in layer 4 and 3, in contrast with the comparison with the lidar and SAGE II which showed a small negative bias. A better agreement is generally found with the new umkehr retrieval in the lowermost layers.

The average difference between the ancillary measurements and each Umkehr retrieval is displayed all together in figure 4, in order to compare the behavior of each data set with respect to the Umkehr one. The comparison with the lidar and the SAGE II measurements are generally very similar, in particular from layer 5 to 8. Very small differences, generally less than 5% are found with both Umkehr retrievals. In layer 4, the average difference becomes negative with the new retrieval in both the SAGE II and lidar cases. In layers 3 and 4, the difference is somewhat larger with SAGE than with the lidar. The average difference with the sondes is systematically positive but the agreement is improved with the new algorithm.

As a conclusion, the comparison between the lidar, SAGE II, the sondes and the Umkehr records shows generally very small differences not exceeding 5% in absolute value from layer 5 to 9. The new retrieval yields less ozone in layer 4 which changes the sign of the bias with the lidar and SAGE data and reduces the bias with the sondes. If one takes into consideration only the comparisons based on a sufficiently large number of events, the higher difference is seen in layer 3 with the new retrieval with a bias with SAGE of -10%.

SAGE - Umkehr comparison at the various stations.

An extensive comparison was performed with the SAGE II data at the 13 Umkehr stations selected for the final Umkehr analysis. The criteria for the comparison were the same as for the OHP. A first glance at the monthly sampling of the SAGE II measurements at the various stations shows that it highly differs from one station to the other (see figure 5, which shows the total number of SAGE II ozone profiles per month at each station during the 15 year period). The best sampling of the ozone field is generally obtained in winter in the northern mid-latitudes stations like Belsk, OHP, Arosa or Sapporo with for instance more than 200 data obtained in January at Arosa over the 15 year period. In summer, the sampling is much less with only 18 measurements in August in the same station. In the subtropical and tropical stations, the best covererage is also in January but it never exceeds a total of 50 measurements over the whole period. In the southern hemisphere in Perth, the best coverage is also obtained in winter in July.

As a consequence, the number of coincidences will vary a lot from one station to the other, depending also on the quality of the Umkehr record.

The average difference between the SAGE II measurements and both Umkehr retrieval is displayed in figure 6, in the same way as for the OHP comparison. In addition, is shown the number of coincidences, which varies with altitude due to the filtering of the SAGE II data after the Mount Pinatubo eruption. One can see that the number of coincidences ranges roughly from 15 in New Dehli to 450 in Arosa. Besides, the number of coincidences can be different depending on the Umkehr retrieval. This is due to different selection criteria between both algorithms (see the section on the quality of the data in the LAP report). This difference is more visible when the number of events is low as in Kagoshima for instance or New Dehli. It gives also an indication on the quality of the Umkehr record, since "good" measurements should pass both selection criteria. In Arosa and OHP for instance, the number of coincidences is very similar.

In most stations with a reliable long term Umkehr record, the new and old algorithms show a similar behavior from layer 5 to 9, with practically no bias in layers 5-8 and a positive bias in layer 9. Note that the results for Arosa and OHP are nearly identical. In layer 3, the old algorithm generally shows a rather large negative bias, followed by a smaller positive bias in layer 4. With the new algorithm, a negative bias is obtained in both layers. In layer 4, the bias is of the same magnitude but it is smaller in layer 3. seen with the new algorithm provides, a negative bias in layer 4. Some stations show a larger positive bias in layer 8, similar with both retrievals like Belsk or Sapporo. In Mauna Loa, Cairo, Naha, a larger negative bias is seen under layer 6. The largest difference between both Umkehr retrievals is seen at Kagoshima, New Dehli, Naha and Aswan.

As a conclusion, the comparison with SAGE II shows generally a somewhat positive bias in the uppermost layers (from layer 7 or 8 depending on the station) and a negative nias in the lowermost layers. In most stations, the difference between both Umkehr retrievals is observed in the lower layers 3 and 4.

Figure 3: Average difference between the old or the new Umkehr retrieval and (1) the lidar measurements, (2) the SAGE II measurements and (3) the ozone soundings

Figure 4: Comparison of the bias between the Umkehr and the ancillary measurements at OHP for the new and the old Umkehr retrieval

Figure 5: Total number of SAGE II measurements per month over the 15 year period.

Figure 6: Average difference between SAGE II and the old and the new Umkehr retrievals at the various Umkehr stations

Figure 6: Average difference between SAGE II and the old and the new Umkehr retrievals at the various Umkehr stations

Figure 6: Average difference between SAGE II and the old and the new Umkehr retrievals at the various Umkehr stations

Figure 6 : Average difference between SAGE II and the old and the new Umkehr retrievals at the various Umkehr stations

## PART B :  INDIVIDUAL REPORT

Reporting period :  February 1998-October 1999

Contractor: Instituto Nacional de Meteorologia, Spain (Izaña Atmospheric Observatory)

Leading scientist: Emilio Cuevas (ecuevas@inm.es)

Scientific Staff: Alberto Redondas (aredondas@inm.es), Juanma Sancho (jsancho@inm.es), Virgilio Carreño (vcarreno@inm.es).

Address: Izaña Observatory, C/ La Marina, 20, Atico, 38071 S/C de Tenerife, Spain.

Telephone: +34-922-373878

Fax: +34-922-373720

I. Objectives for the reporting period.

The main achieved objectives of the reporting period are the following:

- Compile the three existing Brewer-Umkehr codes (Wave5-old, Wave5 and Wave6) in 32 bits.

- Re-evaluate the three different Brewer-Umkehr codes comparing their results with ECC vertical ozone profiles for the periods when ozone sonde intensive campaigns were performed in the framework of REVUE project (July 1996 and July 1997), testing Umkehr quality control criteria, and taking into account the following improvements and corrections:

- As Izaña station is at 2367 m .a.s.l., layers 0 and 1 are considered as an unique layer 1, in which the ozone integrated from sea level to Izaña’s altitude is subtracted.

- ECC vertical profiles were corrected by the factor Brewer-O3/ECC-O3 and integrated for the different Umkehr layers (DU).

- A new firts-guess based on the ECC vertical ozone climatology of Izaña was incorporated for layers 1-7. For upper layers the new first-guess assumes a constant ozone mixing ratio.

- Different values in the parameters controlling the simple and multiple scattering (IPNOT and PFACT, respectively) were tested for the different codes, and their influence on the Umkehr profiles were evaluated.

- Modifications in the ozone covariance matrix were tested.

- Re-evaluate and analyze the whole Izaña's ECC and Brewer-Umkehr data set corresponding to the period 1992-1998. Seasonal variations and outstanding features of the Umkehr time series are analyzed.

# II. Main results obtained

The main results obtained in the REVUE project by INM group since 1996 are shown in the following two reports:

- "Analysis and validation of the different Brewer-Umkehr codes", by A. Redondas, E. Cuevas, V. Carreño and J. Sancho.

- "Umkehr long-term analysis at Izaña Atmospheric Observatory", by E. Cuevas, A. Redondas and J. Sancho.

#### III. Main conclusions

Concerning the Brewer-Umkehr code validation:

- Wave5-old, Wave5 and Wave6 Brewer-Umkehr codes have been carefully analyzed and validated against ECC vertical profiles. The most simple Umkehr code, the early version Wave5 (1991 code) provides reasonably good results. Wave5 and Wave6 (1994 codes) do not seem to improve Umkehr profiles and, on the contrary, might introduce some uncertainties.

- An analysis of possible Umkehr quality control indexes has been performed. The cloud monitoring, the number of iterations, the RMSRES and the 303.6/323.2 ratio against the zenith angle, provide helpful information about the Umkehr success.

- The PFACT parameter that controlls the multiple scattering, and is incorporated in wave5 and wave6 codes, seems not to be much useful, at least for the Izaña Observations.

- A new ozone first-guess obtained from the ECC climatology for 1-7 layers has been incorporated to Wave5 (1991 code), improving significantly the results.

Concerning the Umkehr long-term analysis:

- The high optical depth recorded during Saharan air mass outbreaks seems not to affect the Umkehr results. This could be so due to the fact that under these situations the well-mixed and hazy Saharan air mass is approximately confined to a 4-km thickness layer between 1 and 5 km altitude.

- The Izaña’s 1992-98 Umkehr series show a reasonable good agreement with ECC series. Most of the errors fall in the 10% range at layers 4, 5 and 6.

- Layer 1 (2.4-10.3) series shows a similar behavior to the surface ozone series at the Izaña Observatory, with a maximum in late spring-early summer and a minimum in late fall.

- Layer 2 (10.3-14.7) shows a maximum in spring. This layer is strongly influenced by the altitude variation of the double tropopause found in subtropics.

- Layer 3 (14.7-19.1 km) shows a minimum in late summer - early fall.

- Layer 4 (19.1-23.5 km) shows a seasonal variation similar to that of total ozone with a maximum in spring. However the annual minimum is observed earlier that in total ozone series (late summer).

- Layer 5 (23.5 -28 km), where the ozone peak is observed in the subtropics, shows also a similar behavior than the total ozone series except that the annual peak is recorded later (early summer).

- Layer 6 (28.0-32.6 km) shows very well the "anomalous" ozone minima recorded in total ozone in the winters 1992-93 and 1994-95.

- Nothing statistically significant can be assessed concerning the ozone trends at different Umkehr layers because the series records are too short (6 years).

Umkehr series at Izaña Observatory

## Cuevas, A. Redondas, J. Sancho and V. Carreño

Izaña Atmospheric Observatory (Instituto Nacional de Meteorología, Spain)

Contribution to the final Report of REVUE ("REconstruction of Vertical ozone distribution from Umkehr Estimates") Project, EC contract:  ENV4-CT95-0161.

1.  Introduction

Umkehr observations today are performed with both Dobson (WMO, 1992; Staehelin et al., 1995) and Brewer (Kerr et al., 1988) spectrophotometers measuring the ratio of diffusely transmitted zenith-sky radiance at a wavelength pair in the ultraviolet, one wavelength strongly, the other wavelength weakly absorbed by ozone. These wavelength pairs are measured in a series of zenith-sky observations with the solar zenith angle changing from 60º to 90º during sunrise or sunset.

The Brewer spectrophotometer is an automated instrument, so most of the Brewer stations report Umkehr measurements, which are inverted and archived as layer ozone amounts at the World Ozone Data Center, Toronto, Canada. Some stations have today data records exceeding 10 years, long enough to contribute at the ozone trend analysis (McElroy et al., 1996). However, the Brewer data require additional scientific effort to invert and validate before they can be used for trend studies. One of the main goals of the Izaña's team in the REVUE Project is to re-process the whole Umkehr dataset obtained at the Izaña Atmospheric Observatory, and validate it against the ECC dataset obtained at the Santa Cruz de Tenerife radio sonde station.

The Izaña Atmospheric Observatory is located at 28º18'N, 16º29'W on the island of Tenerife (Canary Islands) at 2367 m.a.s.l. The Observatory is managed by the Instituto Nacional de Meteorología (INM), Spain. The station is situated on the top of a mountain plateau on the dorsal ridge that crosses the island. The Observatory is normally above a temperature inversion layer, generally well established over the island (between 1200 and 1800 m a.s.l.), so air is free from local anthropogenic source influences and also from clouds resulting to be extremely clean and suitable for radiation measurements.

Continuous monitoring of total ozone started in May 1991 with a Brewer spectrophotometer (#33). In November 1998 this instrument was replaced by a double Brewer (#157) that is operating since then. The Brewer spectrophotometer has been ozone calibrated at least once a year, since 1991, against the reference Brewer from AES (Canada). The Izaña's Brewer has also successfully participated in four international UV-ozone intercomparisons: NOGIC-93 campaign, the 12th Dobson calibration and intercomparisons (1994), CASCUM-95 campaign (1995), NOGIC-96 campaign and SUSPEN campaign (1997).

The program of vertical ozone profiles (from 0 to about 34 km altitude) was initiated in November 1992 (ECC electrochemical sondes) with ozonesondes launches from Santa Cruz de Tenerife station (36 m.a.s.l.), distant 28 Km. from Izaña Observatory, on a weekly basis and more often during intensive campaigns. The main features of the ozone sonde system are the following:

## Sensor: ECC-A6

Radiosonde: RS-80 15GE

Interface: RSA 11

# Balloons: TOTEX  TA 1200

Receiver: Digicora Marwin MW-11

Wind system: GPS

Data acquisition system: Väisäla Metgraph with NASACONV.

Tenerife station has participated in annual THESEO campaigns and in field experiments carried out in the framework of different EC projects (BOA, CRISTA and TRACAS).

The ozonesondes are checked before launching with a Ground test with ozonizer/test unit TSC-1. Column ozone comparisons are done primarily using the Brewer located at Izaña, and secondarily with TOMS. Now the NDSC quality control and quality assurance requirements are fulfilled, and the ozone sonde data are routinely archived into the NILU database.

The location of the station is favorable to perform studies of chemical and transport processes in the stratosphere of the subtropics and evaluate their importance for mid-latitudinal ozone levels.

In this paper the Umkehr and ECC series obtained at Izaña Observatory in the period 1993-98 are showed and compared each other. Wind information derived from the sondes is also provided for each Umkehr layer.

2. Experimental

Confident Umkehr profiles are obtained with the Brewer#33 at Izaña since September 1992. In August 1998 a new double Brewer spectrophotometer (#157) was installed at Izaña which was running in parallel with the older Brewer #33 until November 1998, when the Brewer#33 was moved to a new Spanish station. The complete Umkehr dataset of Izaña Observatory corresponding to the period November 1992 - December 1997 was re-evaluated using the Wave-5 Brewer Umkehr code and the first guess obtained from the ECC climatology. Details of their validation can be found in the accompanied paper by Redondas et al.(1999).Those Umkehr’s fitting the quality control criteria (number of iterations "Iter"  3, and root-mean-square fit between observed Umkehr and Umkehr calculated from the retrieved profile, "RSMRES"  1) were incorporated to the time series.

Information concerning technical features and observation program capabilities with a Brewer Spectrophotometer can be found in McElroy et al., 1989, and in WMO/GAW, 1990.

Concerning the ozone sondes, a constant mixing ratio above burst level is assumed for the determination of the residual ozone if an altitude equivalent to 17 hPa has been reached. The integrated ozone column, for those ozone sondes that reach a burst level of at least 17 hPa, is ratioed with the same column amount measured with Brewer (starting at 2400-m a.s.l.). Most of the Brewer/ECC coefficients fall in the 0.95-1.05 range. The complete ozonesonde dataset of Tenerife station corresponding to the period November 1992 - December 1997 was re-evaluated normalizing them, first, with the factor Brewer-O3/ECC-O3, and integrating them for the different Umkehr layers (DU). As Izaña station is at 2367 m .a.s.l., layers 0 and 1 are considered as an unique layer 1, in which the ozone integrated from sea level to Izaña’s altitude is subtracted. This amount is about 10 DU (Redondas et al., 1997). The ECC-Umkehr validation can be only performed for the first seven layers.

3. Umkehr series. Discussion

The ozone first-guess used in the re-evaluation of the Izaña´s Umkehr dataset was obtained from the ECC climatology of Tenerife, as it is recommended by Redondas et al. (2000). This first-guess uses four climatological profiles corresponding to the ozone sondes performed in December-February, March-May, June-August and September-November periods, respectively, from 1993 to 1998 in Tenerife. These profiles (shown in Figure 1) provide information of the vertical ozone distribution from sea level to 32 km. For altitudes higher than 32 km a constant mixing ratio is assumed. An Umkehr-layer scheme in the altitude range where the ECC-Umkehr comparison is performed is shown in Figure 2. The lines correspond to the ozone and temperature vertical profiles in summertime.

A picture of the first guess for the whole period January 1993- December 1997 is shown in Figure 3. In Figures 4a and 4b, the AM and PM January 1993- December 1997 Umkher series with the quality control criteria RMSRES<1 and ITER<=3, using the Wave-5 (1991) code and the ECC-climatology First Guess are shown, respectively. The number of "good" Umkehr profiles in this period is 912, what means that about 50% of the Umkehr profiles obtained every year fit the quality control criteria and are useful to be included in the Umkehr data series of the Izaña Observatory. Finally, Figure 4c shows the ECC 1993-1998 series.

Figure 5a shows the integrated ozone (tropospheric ozone) at Layer 1 (2.4-10.3 km) for AM Umkehr (red crosses) and ECC (blue open circles). Surface ozone series (Dasibi and TECO analyzers) at Izaña Observatory in black crosses is also plotted. Integrated ozone at layer 1 shows a similar behavior to the surface ozone series at the Izaña Observatory, with a maximum in late spring-early summer and a minimum in late fall what seems to be a reasonable result. It is worth to emphasize that the positive trend observed in the surface ozone series is also detected in the Umkehr layers 3 and 4.

In Figure 5b the integrated ozone at Layer 2 (10.3-14.7 km) for AM Umkehr and ECC is depicted. Total ozone is also plotted in black crosses. This layer, that shows a maximum in spring, is strongly influenced by the seasonal variation of the double tropopause altitude in the subtropics (Sancho et al., 2000a).

Figure 5c shows the same as Figure 5b but for Layer 3 (14.7-19.1 km). This layer shows a minimum in late summer - early fall.

In Figure 5d is depicted the Layer 4 (19.1-23.5 km), which shows a seasonal variation similar to that of total ozone with a maximum in spring. However the annual minimum is observed earlier that in total ozone series (late summer).

Layer 5 (23.5-28.0 km), where the ozone peak is observed in the subtropics, is plotted in Figure 5e. This layer shows also a similar behavior than the total ozone series except that the annual peak is recorded later (early summer).

Finally the Layer 6 (28.0-32.6 km) is shown in Figure 5f. This layer shows very well the "anomalous" ozone minima recorded in total ozone in the winters 1992-93 and 1994-95.

None statistically significant conclusion can be assessed concerning the ozone trends at different Umkehr layers because the data series are too short (6 years), and the negative trend in total ozone in the tropics, in the North hemisphere, is rather small: -2.41.5% per decade according to Bojkov y Fioletov (1995). However, an outstanding feature is that layers 3, and 4 show relative ozone recovery since 1995 as it is observed in the total ozone series. In this sense we can say that Umkehr series are sensitive enough to detect ozone trends.

As it can be derived form Figure 6, most of (Umk-ECC)/ECC errors in the period January 1993 - December 1998 are confined into the  10% range for the Umkehr layers 4, 5 and 6, respectively. So, a relatively good agreement between Umkehr and ECC is found in these layers where Umkehr represents very well the actual seasonal variation of ozone. More than 360 quasi-simultaneous Umkehr-ECC profiles have been used in this analysis.

In figure 7, the integrated ozone, the averaged zonal wind, the meridional motion and the wind velocity at layers 3 and 4, respectively are depicted for the period 1994-1999. It can be seen that low ozone is observed when the Eastward component of the wind is higher (around summertime). When this happens the meridional motion (positive values) is also higher, what means that the poleward transport from the equator in the lower stratosphere reduces subtropical total ozone. This fact is particularly clear in the layer 4 (19.1-23.5 km). This result agrees those obtained by Hofmann et al. (1996) who found very low ozone mixing ratio at the tropics below 25 km during poleward transport.

Main conclusions

More than 360 quasi-simultaneous Umkehr-ECC profiles have been compared in the period January 1993 - December 1997, covering all the seasons and different meteorological conditions. So, this Intercomparison and validation can be considered as representative of all type of atmospheric conditions observed in the subtropics.

A detailed analysis of the Umkehr series is out of the scope of our contribution to REVUE Project. However it has been demonstrated in this report that Brewer-Umkehr algorithm is reasonably good in layers 4, 5 and 6 to describe the seasonal variation of ozone in these layers and also to detect trends. Layers 4, 5 and 6 represent together the major part of the ozone layer, so it can be confirmed that Umkehr series, obtained with the Brewer spectrophotometer, are a powerful and helpful tool for ozone layer monitoring. The Umkehr program complements the scarce information provided by the ozonosonde program (because its weekly frequency) about the stratospheric ozone. Indeed Umkehr observations seem to be capable to detect ozone trends.

Bojkov y Fioletov (1995) report negative trend in total ozone of -2.41.5% per decade over the tropics. Sancho et al. (2000b), analyzing a 18-year record of radiosonde data from 1980 to 1998,  observe that stratospheric temperature trends are negative over Tenerife for all levels with a trend lower than -1K/decade between 15 km and 30 km altitude.  Sancho et al. (2000a) also report a positive trend (+131  80 m () per decade)

of the tropical tropopause altitude. Umkehr series will play an important role in interpreting these changes observed in the subtropical stratosphere and so, efforts in Umkehr algorithm improvement will focus on layers 3, 4, 5, and 6 in order to assure an accurate monitoring of the ozone layer in this region. This job can be performed easily in this Observatory since about 50% of the total Umkehr profiles (about 182 Umkehr per year) fit the quality control criteria and are good enough to be included in the Umkehr data series for statistical analysis.

Near future work concerning ozone interpretation, in terms of atmospheric transport, will consider the regular two-daily meteorological information (temperature and wind) provided by the meteorological soundings performed in Santa Cruz de Tenerife station and the complete Umkehr daset until now.

Acknowledgments

The authors are especially grateful to Dr. Tom McElroy (AES, Canada) for providing crucial information of the Brewer Umkehr code.

The authors thank Sergio Afonso, field technician of the ozone sonde program at the Izaña Observatory.

##### References

Cuevas, E., J. Sancho, A. Redondas, Investigaciones sobre Ozono Troposférico en el Observatorio de Vigilancia Atmosférica de Izaña, Física de la Tierra, nº 9, 67-106, Servicio de Publicaciones de la Universidad Complutense, Madrid, Editorial Complutense, 1997.

Bojkov, R.D., V.E. Fioletov, Total Ozone Variations in the Tropical Belt: An Application for Quality of Ground Based Measurements, Meteorol. Atmos. Phys., 1995.

Hofmann et al. Geophys. Res. Lett., 23, 1533, 1996

Kerr, J.B., I.A. Asbridge, and W.F.J. Evans,  Intercomparison of total ozone measured by the Brewer and Dobson spectrophotometers at Toronto, J. Geophys. Res., 93, 11,129-11,140, 1988.

McElroy, C.T., C.L. Mateer, J.B. Kerr, and D.I Wardle, 1989, Umkehr Observations made with the Brewer ozone Spectrophotometer, Ozone in the Atmosphere, A. Deepak Publishing, 1989.

McElroy, C.T., J.F. Hahn, and E. Hare, Determining high-altitude trends in ozone from Brewer Umkehr observations made at Canadian stations, Proceedings of the Quadrennial Ozone Symposium, 25-28, 1996.

Redondas, A., E. Cuevas, J. Sancho, Investigaciones sobre Ozono Estratosférico y Radiación Ultravioleta en el Observatorio de Vigilancia Atmosférica de Izaña, Física de la Tierra, nº 9, 29-66, Servicio de Publicaciones de la Universidad Complutense de Madrid, Editorial Complutense, 1997.

Redondas, A., E. Cuevas, V. Carreño, J. Sancho, Analysis and validation of the different Brewer-Umkehr codes, Contribution to the final Report of REVUE ("REconstruction of Vertical ozone distribution from Umkehr Estimates") Project, EC contract: ENV4-CT95-0161, February 2000.

Sancho, J.M., E. Cuevas, V. Carreño, Characterization of the Double-Tropopause over the Subtropical Region from 1981-1996 Radiosonde Observations at Tenerife, Proceedings 2ª Asamblea Hispano-Portuguesa de Geodesia y Geofísica, Lagos (Portugal), 8-12 February, pp 455-456, 2000a.

Sancho, J.M., E. Cuevas, J. Rodríguez, Long-term changes in tropospheric and stratospheric temperature profiles over Tenerife, Proceedings 2ª Asamblea Hispano-Portuguesa de Geodesia y Geofísica, Lagos (Portugal), 8-12 February, pp 535-536, 2000b.

Staehelin, J., H. Schill, B. Högger, P. Viatte,  G. Levrat, and A. Gamma,  Total ozone observation by sun photometry at Arosa, Switzerland, Opt. Engin., 34, 1977-1986, 1995.

WMO/GAW, WMO Consultation on Brewer Ozone Spectrophotometer Operation, Calibration and Data Reporting, 1990.

WMO, Handbook for Dobson Ozone Data Re-evaluation, World Meteorological Organization Global Ozone Research and Monitoring Project — Report No. 29, Geneva, 1992.

##### Figures captions

Figure 1. Averaged vertical profiles of temperature and ozone partial pressure corresponding to spring (March-May), summer (June-August), fall (September-October) and winter (December-February), respectively. These mean profiles were calculated with the ozone sondes performed in the period November 1992 - December 1997.

Figure 2; Umkehr layer scheme in the altitude range where the ECC-Umkehr comparison is performed. The lines correspond to the ozone and temperature vertical profiles in summertime.

Figure 3; Picture of the ozone first guess using the ECC climatology of Tenerife station for the period 1993-98.

Figure 4; Time-Layer cross section of AM and PM Umkher and ECC for the period 1993-98. The Umkehrs used in this plot were re-evaluated using the Wave-5 (1991) Umkehr code, the ECC-climatology ozone first guess, and all of them fit the quality control criteria (RMSRES<1 and ITER<=3).

Figure 5 (a, b, c, d and f); Plots of AM Umkehr (red crosses) and ECC (blue open circles) for Layers 1, 2, 3, 4, 5 and 6, respectively. Surface ozone (Figure a) and total ozone (Figures b, c, d, e and f) in black crosses.

Figure 6 (a and b); (Umk-ECC)/ECC errors for the Umkehr layers 4 and 5, respectively, in the period January 1993 - December 1998.

Figure 7 (a, b); Integrated ozone (Blue line). Averaged zonal wind (u - black line-; positive values mean Eastward component), meridional motion (v; - red line --; positive values mean Northward component; ) and wind velocity (V; green line) at layers 3 and 4, respectively. Period: 1994-1999.

Aerosol Climatology Provided to the Boulder Team

Contractor: 		I.R.O.E.-C.N.R

Leading Scientist:	 Ing. Leopoldo Stefanutti

Scientific Staff:		Ing. Leopoldo Stefanutti

Dr. Donatella Guzzi

Mr. Marco Morandi

Dr. Vincenzo Santacesaria

Address: 	Via Panciatichi 64, 50127, Florence, Italy

Telephone:	+39 55 42351

Fax:		+39 55 410893

Telex: 		570231 IROE I

E-Mail:		moana@iroe.iroe.fi.cnr.it

lidar@iroe.iroe.fi.cnr.it

guzzi@iroe.iroe.fi.cnr.it

santac@iroe.iroe.fi.cnr.it

I  Aerosol climatology provided to Boulder team:

Within the framework of the REVUE project, IROE provided stratospheric aerosol climatology using Lidar data collected at the sites of Sodankyla (67o N  26.50o E), Lauder  (45o S   170o E), Dumont d’Urville  (66.41o S  140o E), Florence (43.5o N    11o E).

Processed stratospheric Lidar data coming from the sites listed above were kept currently updated. Related optical properties of stratospheric aerosols have been used for a statistical analysis.

For the evaluation of aerosol effects during the retrieval of ozone profiles in both the stratosphere and the troposphere, IROE has also supplied Boulder group with integrated aerosol backscattering coefficient and optical depth for all NDSC Lidar stations at which this kind of information is available, that is to say: Garmish-Partenkirken (Germany), Observatoire Haute Provence (France), Mauna Loa, MacMurdo (Antarctica), Eureka and Thule (Arctic). In addition to NDSC stations, data coming from other no-NDSC Lidar stations (i.e. Thessaloniki-Greece), together with NMC information for the July/August and December/January periods was given for the listed above NDSC sites.

II	Main results obtained:

The processing of the data coming from the Lidar sites of Dumont d’Urville, Sodankyla, Lauder and Florence has been updated to 1998.

As a final product, a large database of aerosol parameters has been produced. The computed parameters (i.e. Aerosol Backscatter Ratio, Backscatter and Extinction Coefficients, and Depolarisation profiles) have been given to Boulder as a function of the geometrical altitude and of the potential temperature for all the cases for which PTU rawindsonde data are available.

In regard to PSC phenomena, on the basis of these measurements for the Antarctic and Arctic sites, the mid-cloud height and temperature, cloud top and bottom, and the optical parameters such as extinction, backscattering and depolarisation for each cloud layer has been given. The evolution of these parameters with time, altitude and temperature are available. In this way, it has been possible to evidence the homogeneity of the cloud: that is to say, whether the cloud is composed of liquid or solid particles (homogeneous cloud) or of a mixture of both solid and liquid ones (non-homogeneous), and its changes in composition with time. This climatological analysis of PSCs events in the Arctic and Antarctic regions, makes it possible to determine the variations in altitude and composition (i.e. solid, liquid or mixed phase) of stratospheric loading during one or several years of measurements.

Fig.1 Seasonal evolution of midcloud PSC    Fig.2 Seasonal evolution of  midcloud PSC altitude,  1989 – 1996                                                         temperature, 1989 – 1996

With regard to the evolution in PSCs during the year, the midcloud altitude and temperature are plotted in fig. 1 and 2. During the winter, an increase in the midcloud temperature and a decrease in midcloud altitude were observed.

PSCs with high SR values were observed during the second half of July (around Julian day 220) and during the entire month of August, as is clearly shown in fig.3. The  midcloud temperature  of these clouds was always below 188K (see fig.4).

| Fig.3 Seasonal Scattering Ratio trend for PSCs at Dumont d’Urville | Fig.4 PSCs Scattering Ratio values vs temperature at Dumont d’Urville |
| --- | --- |

In both polar and non-polar sites of the IROE Lidar network, the long-term monitoring of stratospheric aerosols has made it possible to compute  the decay time of aerosol loading, in both the northern and southern hemispheres. The aerosol decay time constant was computed by taking into account only the measurements performed when the polar sites were out of the polar vortex and it was obtained by linearly interpolating the logarithm of the integrated backscattering value. Some results are shown in table I, where the decay constant evaluated for the period 1.1.1994-31.12.1996 and 1994-1997 is reported.

| ## AEROSOL DECAY TIME CONSTANT |  |  |  |
| --- | --- | --- | --- |
| ## Sodankyla (Finland) | ## Lauder ## (New Zealand) | ## Firenze ## (Italy) | ## Dumont d’Urville (Antarctica) |
| 2.90±0.50 years (a) | 2.80±0.85 years (a) | 2.96±0.66 years (a) | 2.29±0.60 years (a) |
| 3.60±0.70years (b) | 3.40±0.95 years (b) | 3.20±0.80 years (b) | 3.05±0.78 years (b) |

Table I: Aerosol decay time constant for the period 1994-1996 (a) and 1994-1997 (b).

The decay time constant is almost the same for all the latitudes, an increase can be observed for the 1994-1997 period, showing a comeback to a situation of background stratospheric aerosol.

Data coming from other Lidar NDSC (Garmish, Observatoire Haute Provence, Mauna Loa, MacMurdo, Eureka, Thule) and no-NDSC stations (Thessaloniki) were all sent directly to John deLuisi’s group in Boulder. In addition to these data sets, NMC data for the period July/August and December/January were also provided.

The backscattering coefficent integrated between 13Km and 25Km since 1989 for several NDSC sites is plotted in figure 5.

In figure 6 the whole Garmish record for the integrated backscattering is shown, signatures of the last volcanic eruptions are visible in the time series.

PTU soundings are available for almost all the lidar measurements. Those data were used for a climatological study that monitors the changes and evolution in time and height of the optical (mean scattering ratio and integrated backscattering) and microphysical properties (aerosol mass content and mean radius) of aerosols above these sites. This analysis would be useful in determining aerosol effects in the retrieval of ozone profiles using the Umker method.

Figure 5 Integrated backscattering for 6 NDSC lidar stations placed in northern and Southern Hemisphere since 1989. The four site of IROE network are included.

Figure 6 Garmsh Partenkirchen time series for integrated backscattering

For better understanding the effect of the aerosol loading in the retrieval of ozone using Umkher method the evolution of the altitude of the aerosol layer above lidar stations was observed. In figure 7 the evolution of the Scattering Ratio was reported for the 4 of the 6-lidar sites used for figure 5.

Figure 7 Evolution of the aerosol layer above OHP, Garmish, Florence/Brasimone and Lauder

It can be noticed that after the eruption of Mt. Pinatubo the height of the aerosol layer first increased reaching an altitude of 21km during 1992, than in the period 1993-1994 the altitude decreased to about 16km, After 1994 the aerosol layer is stable at an altitude between 17-18 km.

Under the assumption that aerosol particles are a homogeneous sphere, the backscattering efficiency may be approximated as a linear function of the particle size parameter , x:

4pQbs(x,n)»C(n)x

where x=r/, r is the particle radius and n is the refractive index of the particle.

Using such an assumption, it is possible to relate the aerosol mass content, M, to the integrated backscattering coefficient , regardless of the dependence on size distribution as follows:

M»8ρλβ(3C(n))

with  is the density of the aerosol droplets and is the wavelength of the incident radiation.

The linear coefficient C (n) is dependent on particle composition. It was evaluated for different refractive index and aerosol particle composition, linearizing the computed Mie backscattering efficiencies with respect to their size parameter.

Steele and Hamill derived the composition of sulphuric acid aerosols as a function of temperature and water vapour pressure. We combined their results with the information coming from the PTU soundings to determine the refractive index and density of aerosol sulphuric acid droplets. These data and the values of integrated backscattering were employed to evaluate the aerosol mass content for all the lidar sites when temperature and humidity information was available.

The changes in the aerosol mass content during the period 1.1.1994-31.12.1996 are plotted in figure 8.

Figure  8 Aerosol mass content for IROE Lidar network since 1.1.1994

Another important aerosol optical characteristic for the revaluation of ozone using new Umkher algorithms is the optical depth. In Florence we have evaluated it at 532nm, the green wavelength of the lidar signal. In figure 9 the changes of the optical depth after 1989, above IROE network sites, are shown.

To better determine the aerosol impact in the ozone retrieval, particle size distributions and characteristic refractive indices for stratospheric and tropospheric aerosols were provided. A characteristic size distribution for tropospheric urban aerosols is a multimodal size distribution, that is the sum of 3 lognormal distributions having the following mean radius: Rmed = 0.0085, 0.0314, 0.083 micron, and standard deviations equal to: s=1.69, 2.14, 3 respectively. While stratospheric volcanic aerosols can be characterised by a lognormal monomodal distribution with Rmed = 0.0725 micron and a standard deviation s = 1.8 and Ntot = 10 cubic cm, or by a Modified Gamma distribution with the following value for the parameters A = 341.33 or 5461.33 cubic cm/ m, alpha  = 1, gamma = 0.5, b = 3 or 4   1/micron.

Figure 9 Aerosol optical depth above IROE lidar stations

Background aerosol size distributions coming from campaign performed in: Antarctica (Mac Murdo), New Zealand (Lauder), USA (Wyoming), France (Observatoire Haute Provence), Sweden (Kyruna) during1994-1997 period, obtained from Terry Deshler optical particle counter, were sent to Boulder group.

The background aerosol is characterised by a bimodal size distribution. No evident changes are noticeable indication of the size distribution obtained between 17-18 km are reported in table II.

17 km

| Year | N1 (cm-3) | R1 (um) | So1 | N2 (cm-3) | R2 (um) | S02 |
| --- | --- | --- | --- | --- | --- | --- |
| 1994 (NZ) | 44.7 | 0.108 | 1.18 | 0.348 | 0.389 | 1.21 |
| 1995 (NZ) | 44.7 | 0.0343 | 1.97 | 0.241 | 0.363 | 1.23 |
| 1996 (NZ) | 44.7 | 0.0594 | 1.78 | 0.000062 | 1.38 | 1.16 |
| 1995 (FR) | 15.4 | 0.0385 | 2.17 | 0.24 | 0.408 | 1.26 |
| 1997 (FR) | 17.3 | 0.0377 | 1.81 | 0.194 | 0.348 | 1.26 |

18 km

| Year | N1 (cm-3) | R1 (um) | So1 | N2 (cm-3) | R2 (um) | S02 |
| --- | --- | --- | --- | --- | --- | --- |
| 1994 (NZ) | 40.1 | 0.0416 | 1.89 | 0.163 | 0.328 | 1.31 |
| 1995 (NZ) | 40.1 | 0.0453 | 1.79 | 0.306 | 0.343 | 1.22 |
| 1996 (NZ) | 40.1 | 0.02 | 2.45 | 0.116 | 0.302 | 1.48 |
| 1995 (FR) | 9.08 | 0.0529 | 2.03 | 0.246 | 0.383 | 1.27 |
| 1997 (FR) | 10 | 0.0579 | 1.94 | 0.000298 | 1.31 | 2.17 |

Table II Changes of the aerosol size distribution coming from Terry Deshler measurements during 1994-1997 period. (NZ = Lauder, New Zealand; FR =OHP, France)

The decay time evaluated for integrated backscattering is also an indication of the dimension of the particles: the larger the size of the particles, the shorter the decay time. The values reported in table I are consistent with the value of the sedimentation velocity for sulphuric aerosol particles, with a mean radius of around 0.3-0.4 microns for the 1994-1996 three-year period, and a mean radius of 0.1-0.2 micron for the 1994-1997 four-year period. Computations for the 1992-1993 two-year period after the eruption led to a time constant of 0.8 year, corresponding to a mean radius of about 0.9 micron. This rough estimate of the mean value of the Pinatubo aerosol radius is comparable to the mode median radius of 0.8 micron (modal width of 1.3) of the substantially monomodal size distribution, characteristic of a post-volcanic eruption stratospheric aerosol loading, obtained by balloon-borne instruments at Laramie during the 1992-1993 period.

Some information on the refractive index for both Tropospeheric and Stratospheric aerosols, listed in table III and IV, were sent to boulder for a better characterisation of the impact of aerosol in the Umkher retrieval.

| Urban Aerosol Refractive Index | City |
| --- | --- |
| 1.43 +/- 0.02 | Los Angeles |
| 1.47 +/- 0.03 | “ |
| 1.53 +/- 0.05 | Calgary |
| 1.57+/- 0.04 | Mainz |
| 1.55+/- 0.02 | Boulder |

Table III: Refractive Index for Tropospheric urban aerosols at 532nm.

| Stratospheric Aerosol Refractive Index | Aerosol Type |
| --- | --- |
| 1.32+/- 0.01 | Ice Clouds |
| 1.43 +/- 0.04 | Volcanic Aerosol |
| 1.39 +/- 0.03 | Cloud Bynary acid solution |
| 1.42 +/- 0.04 | Cloud Ternary acid solution |

Table IV: Refractive Index for Stratospheric aerosols at 532nm.

III	CONCLUSIONS

IROE’s main task was to provide the most extensive information possible on the optical properties of aerosols. Our data were used to improve Umkher algorithms developed by other patners.

The aerosol climatology was provided first of all by using data coming from our Lidar stations located in both the northern and southern hemispheres. Data coming from Lidar NDSC and no-NDSC sites were collected and used to increase knowledge on the global aerosol climatology.

The aerosol effect in the troposphere was considered providing PBL data from two Lidar stations, one in Florence and the other in Thessaloniki.

IV  LIST OF PUBLICATIONS:

-D. Guzzi, M. Morandi, V. Santacesaria, L. Stefanutti, P. Agostini, V. Cuzzola:. “ Stratospheric Aerosol Climatology in North and South Hemisphere”, 19th International Laser Radar Conference, 6-10.7.1998, pg 501-504 .

- D.Guzzi, L.Stefanutti, Francesco Castagnoli, Marco Morandi, Vincenzo Santacesaria: “ Six Years Aerosols and PSCs  Measuraments in Dumont D’Urville (Antarctica)”, VI Workshop on Antarctic Atmosphere, 21-23.10.1997, pg. 295-306

- D. Guzzi, L. Stefanutti, V. Sanatcesaria, M. Morandi, P. Agostini, J.P. Wolf, B. Liley: “ Four years of stratospheric aerosol measurements in the northern and southern hemispheres”. Geophysical Research Letter, Vol. 26, n.14, pg 2199-2202, 1999.

# - V. Santcesaria, L. Stefanutti, D. Guzzi, R. Mackenzie: “ Polar Stratospheric Cloud climatology (1989-1997) from LIDAR measurements over Dumont d’Urville (Antarctica)”, TellusB, ,2000 (In corso di pubblicazione)

- D. Guzzi, M. Morandi, L. Stefanutti, P.Agostini, B.Liley: “ Stratospheric aerosol measurements in the polar regions of the  northern and southern hemispheres”, VIII Workshop on Antarctic Atmosphere, 20-22.10.1999 (in stampa)

References

? Pinnick, R.G. , S.G. Jennings, P. Chylek, “Relationship between extinction, absorption, backscattering and mass content of sulfuric acid aerosols”, J. Geoph. Res. Vol. 85, pp.4059-4066, 1980

Guzzi D. , “ FORTRAN programs for the determination of the scattering properties of spherical particulate”, IROE Internal Report n. RR/POE/93.18, 1993

Steele, H.M., P. Hamill, “ Effects of temperature and humidity on the growth and optical properties of sulfuric acid water droplets and Stratosphere”, J. Aerosol Sci., Vol. 12, pp. 517-528, 1981